3GPP SA WG2 Meeting #161 	S2-24xxxxx

26 Feb - 1 Mar, 2024, Athens, Greece	(revision of S2-2400698)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | draft | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23501 draft CR for 5GC support of IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ 5GC_ IoT NTN |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specific aspects for Cellular IoT over satellite access into 5GC are not standardized |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describes the specific aspects for Cellular IoT over satellite access into 5GS. - New IoT NTN RAT types (as defined in clause 4.13.2 in TS23.401) are introduced for 5GC to identify and restriction for the different IoT satellite access. - Reusing the mechanisms and relevant chapters designed for cellular IoT in 5GC and NR NTN in 5GC for support of IoT NTN in 5GC. These mechanisms includes network selection, tracking area handling, mobility registration update, mobility forbidden area, service area restrictions, discontinuous coverage, verification of UE location. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC support of IoT NTN is not supported and satellite operators deploying global service will miss the oportunity of opting for a single-core solution for both NR NTN and IoT NTN which will offer distinct advantages in reducing both CAPEX and OPEX from a cost-efficiency standpoint, while promoting integrated IoT-NTN and NR-IoT integrated deployment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.a(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change(All text is new) * * * *

5.4.11a	Support for Cellular IoT over satellite access into 5GS

This clause describes the specific aspects for Cellular IoT over satellite access into 5GS.

Support of network selection for cellular IoT connect 5GC as described in 5.31.3 applies to IoT NTN connect 5GC.

Unless otherwise stated in this specification, the following clauses for support for NR satellite access into 5GS also apply to Cellular IoT over satellite access into 5GS:

-    5.4.10 Support for identification and restriction of using NR satellite access

-    5.4.11 Support for integrating NR satellite access into 5GS, including Network selection, tracking area handling, mobility registration update, mobility forbidden area, service area restrictions

-    5.4.13 Support of discontinuous network coverage for satellite access

RAT Types values "WB-E-UTRAN(LEO)", "WB-E-UTRAN(MEO)", " WB-E-UTRAN(GEO)", " WB-E-UTRAN(OTHERSAT)", "NB-IoT(LEO)", "NB-IoT(MEO)", "NB-IoT(GEO)", "NB-IoT(OTHERSAT)", "LTE-M(LEO)", "LTE-M(MEO)", "LTE-M(GEO)" and "LTE-M(OTHERSAT)" as introduced in EPC for satellite access in clause 4.13 of TS 23.401, are used in 5GC to identify and restriction for the different E-UTRA satellite access types.

The AMF determines the RAT Type for E-UTRA satellite access. When the UE is accessing E-UTRA using satellite access, an indication is provided in N2 interface indicating the type of E-UTRA satellite access, as defined in TS 38.413 [34].

Support for Cellular IoT over satellite access is specified in TS 38.300.

Support of verification of UE location for NR NTN connect 5GC as described in 5.4.11.4 applies to IoT NTN connect 5GC.

NOTE 1: 	There is no specific support for network-initiated location verification for Cellular IoT over satellite access in this release.

NOTE 2: 	The specific support for UE Coarse location reporting for NB-IoT over satellite access is not considered in this release. UE Coarse location reporting for NB-IoT over satellite access to EPC subjects to the progress in Rel 18. When the solutions are agreed, the solutions can be mapped to NB-IoT over satellite access to 5GC.

# * * * * End of changes * * * *