3GPP TSG-SA WG2 Meeting #161	S2-2402429

Feb 26 – Mar 01, 2024; Athens, Greece	(revision of S2-2400301r01)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1378 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network initiated IMS DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to LS from GSMA UPG (S2-2400034), in some cases, UE may not be pre-provisioned to initiate IMS Data Channel so the network-initiated Bootstrap DC setup capability is necessary for supporting those UEs. In some cases, e.g customer calsto retailer’s Sales Support or to the Customer Care number and service representative or automatic IVR wants to initiate interactive menu during the call. In currest spec, the Bootstrap DC setup procedure is initiated by the UE only. Therefore, the case that IMS AS triggers this procedure needs to be further described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The description that the network (e.g IMS AS) can upgrade an ongoing IMS audio/video session through a SIP re-INVITE containing bootstrap data channel establishment request is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Bootstrap DC setup procedure cannot be initiated when the UE is not pre-provisioned when to initiate IMS Data Channel. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AC.4	IMS DC Channel Setup

The following principles apply when an IMS Data Channel is established: -

-	UE can establish an IMS Data Channel simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to an IMS Data Channel session.

NOTE:	An IMS Data Channel established simultaneously with an IMS audio/video session can have impact to the IMS session setup time.

-	The network (e.g IMS AS) can upgrade an ongoing IMS audio/video session through a re-INVITE by including SDP offer for the bootstrap data channel establishment request to an IMS Data Channel session. The IMS AS triggers this procedure after validating user subscription data that the UE supports IMS data channel and the UE context which indicates the IMS DC capability supported by UE.

NOTE:	When the IMS AS triggers the above procedure is not defined in this release.

-	The UE may be configured by the HPLMN either via Device Management or in the UICC whether and when to initiate a DC establishment request. If the UE is not configured by the HPLMN, it is left for UE implementation whether and when to initiate the DC establishment request. The UE shall not initiate a DC establishment request if the network has not indicated support for IMS DC service during IMS registration or if the UE is configured to not establish an IMS Data Channel.

-	If a UE that has not subscribed to IMS Data Channel establishes an IMS Data Channel simultaneously with an IMS audio/video IMS session, the IMS AS shall discard the Data Channel request and proceed with the audio/video IMS session establishment.

-	The DCSF provides stream ID and a corresponding URL to the MF or MRF via the IMS AS during data channel media resource reservation for the bootstrap data channel that enables downloading a subscriber specific graphical user interface to the UE via the bootstrap data channel. The stream ID is optional. Once the MF or MRF, acting as HTTP proxy, receive a HTTP GET Request containing the root ("/") URL through the bootstrap data channel, the MF or MRF replaces the root ("/") URL in the HTTP GET Request with the subscriber specific URL corresponding to the stream ID (if available) of the bootstrap data channel and forward the request to the DCSF for downloading the subscriber specific graphical user interface to the UE. The MF retrieves the stream ID of the bootstrap data channel from the transport layer of the data channel connection. The UE shall maintain the HTTP session state when interacting with the MF.

# * * * * End of changes * * * *