3GPP TSG-SA WG2 Meeting #161	S2-240xxxx

26 Feb – 01 Mar 2024, Athens, Greece

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | draft | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE assisted for Enforceable URSP rules provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_UA_UEPO |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on TEI19 WID proposal in S2-2402460, it is proposed to enhance UE Configuration Update procedure to enable UE assisted for Enforceable URSP rules provisioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to enable this new feature, introduce the following: - The network indicates to the UE to report unsupported component(s) or component value(s) in traffic descriptor of a URSP rule in UCU procedure. - The UE provides the unsupported traffic descriptor component(s) or traffic descriptor component value(s) in a UL NAS transport message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary signaling overhead and waste of UE power consumption |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of changes (all new texts)*******

#### 6.6.2.X	Support of URSP status report for traffic descriptor

This clause defines how the network requests URSP status report for traffic descriptor and how the UE reports URSP status report to PCF so that PCF can be made aware what traffic descriptor component(s) or component value(s) are not supported and what actions the PCF may trigger upon the reception of such URSP status report.

When the PCF determines to provision UE policy using UE Configuration Update procedure to UE, it can also indicate URSP status report indication along with UE policy container to the UE. The URSP status report indication indicates to UE to send URSP status report for traffic descriptor. Accordingly, the UE sends the USRP status report for traffic descriptor includes a list of unsupported component type(s), or component value(s) to the PCF via AMF. Based on the USRP status report for traffic descriptor, the PCF can update and provisions UE policy information with enforceable URSP rules.

*******End of changes*******