SA WG2 Meeting #S2-161	S2-2402586

26 February - 1 March, 2024, Athens, Greece	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#1 and 3: New solution on the User profile structure, exposure, link with subscription.

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This paper proposes a new solution to KI#1 and 3 on the User profile structure, exposure, link with subscription.

1. Proposal

It is proposed to accept the change to 23.700-32.

>>>> Start of Change <<<<

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> |
| #X | x |  | x |  |

>>>> Next Change <<<<

6.X	Solution #X: Identifying the Human User of UE based on User Identity Profile stored in the UDM

### 6.X.1	Key Issue mapping

This solution addresses Key Issue #1 and Key Issue #3.

Especially to focus the following bulletins in the Key issue#1 and #3:

-	what information is stored as part of the User Identity Profile (e.g. a User Identifier, associated security credentials, associated devices, user specific QoS settings). Including how User Identity Profiles are created/acquired, stored, and updated,

-	whether and how User Identifiers are linked and unlinked (i.e. associated) with 3GPP subscriptions in an operator-controlled manner, and

-	what and how User Identity Profile information and functionality are exposed (e.g. exposure of the content of the User Identity Profile, exposure of authorization/authentication results, authenticating users, and linking a User Identifier with a 3GPP subscription).

### 6.X.2	Functional Description

In this solution, it is assumed that a User Identifier is unique at least within the operator's network. After authentication of User Identifier is completed, the AMF gets the SUPI of linked 3GPP subscription.

There is User Identity Profiles stored in the UDR. For each User Identity Profile, it may contain following information:

|  | Category |
| --- | --- |
| A User Identifier |  |
| Qos setting | Optional |
| Service information | Optional |
| 3GPP subscription Link list (SUPIs) |  |
| credential of the User Identifier | Optional |

NOTE: 	For each User Identity Profile, it is assumed that there is only one user identifier.

For the UE subscription data, it adds a new IE, active User Identity Profile. It means the user identifier in the User Identity Profile is active with the UE subscription.

The high level principle/assumption of this solution can be summarized as follows:

-	The AF can request 5GC to create/update/delete the User Identity Profile via enhanced Parameter Provisioning procedure.

-	The AF provides the GPSI/SUPI, User Identifier and other information (e.g. Qos setting, service information etc) to create the User Identity Profile.

Editor's note:	The credential of the User Identifier is FFS. It may be determined by SA3.

-	After the user identifier has been authenticated and authorized to use a linked subscription (SUPI) to access the 5GS, the User Identity Profile is marked as the active User Identity Profile in the UE subscription data.

### 6.X.3	Procedures

Figure 6.X.3-1: Provisioning of User Identity Profile

1.	The User interact with AF to create the User identifier.

2.	The AF request NEF to create/update/delete User Identity Profile. The AF may invoke Nnef_PrameterProvision_Create/Update/Delete service operation with GPSI (i.e. Link to this subscription) , User identifier and other information (e.g. Qos setting, service information etc) .

3.	If this request is authorized by the NEF, the NEF request UDM to create/update/delete User Identity Profile. The NEF may invoke Nudm_PrameterProvision_Create/Update/Delete service operation with the parameters received in the step 2.

4.	The UDM requests the UDR to create/update/delete the User Identity Profile.

5.	UDM send the Nudm_ParameterProvision_Create/Update/Delete Response to NEF.

6.	NEF send the Nnef_ParameterProvision_Create/Update/Delete Response to AF.

Editor's note:	The procedure for the User identifier Activation is FFS.

### 6.X.4	Impacts on existing services, entities and interfaces

UDM:

-	create/update the User Identity Profile.

-	stores of User Identity Profile to UDR.

NEF:

-	support provisioning of User Identity Profile.

Editor's note:	This impact to other NF is FFS.

>>>> End of Change <<<<