SA WG2 Meeting #161	S2-2402713

Athens, Greece, 26th February – 1st March 2024

Source: 	CATT, vivo

Title: 	Update the Solution2 for KI#1: TNL connection optimization over N2/S1-AP interface

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes to update the solution 2 for KI#1.

1 Discussion

In TR23.700-29 v0.3.0, solution #2 for KI#1 and solution #4 are proposed a similar solution about TAI/CGI management over N2/S1-AP interface for regenerative-based satellite access.

So it is proposed to merge the TAI/CGI management part into the solution #4 and keep the TNL connection sespend/resume procedure in the solution#2.

2 Proposal

It is proposed to update the solution #2 for KI#1 of TR 23.700-29.

******************** 1st Change ********************

6.2	Solution #2 for KI#1: NG/S1 resumption/suspension for Regenerative-based satellite access

6.2.1	Description

This is a candidate solution for Key Issue #1: Support of Regenerative-based satellite access considering the following aspects:

-	How does the RAN node connect to an AMF on the ground, including what information will be exchanged between RAN and CN?

-	How to efficiently manage N2 connections due to the RAN node’s movement?

Figure 6.2.1-1: Connection status between one satellite (RAN onboard) and AMF(s) via earth station(s) when the satellite is orbiting

RAN node mobility, in case RAN nodes are boarded on satellite and the CN on the ground and RAN orbits the Earth, as shown in Figure 6.2.1-1, it will have regular connection/disconnection with gateways. As gateways support IP connectivity with AMF(s), RAN may have regular connection/disconnection with AMF(s). There are 3 cases considering the connection relationship among gNB, gateway, and AMF.

-	Case I: gNB connects to the same AMF via the same gateway.

-	Case II: gNB connects to the same AMF via a different gateway (e.g. Annex A, Figure A-3).

-	Case III: gNB connects to a different AMF via a different gateway (e.g. Annex A, Figure A-4).

Whether the same AMF/MME is used or not, when the RAN is orbiting away, it will cause much frequent disconnection and re-connection of TNL (Transport Network Layer) over the N2/S1-AP interface of an AMF/MME. When the SCTP endpoint detects the path failure based on heartbeat time out, it will close the SCTP association and delete the transmission control block, which means that, when the satellite flies back to the PLMN A, it has to re-establish the SCTP association via four-way handshake. If all satellites need to re-establish the TNL toward the AMF, there will be lots for signaling towards the AMF on the ground.

The solution proposes to suspend the TNL between the RAN node and the CN if the RAN node attempts to disconnect the backhaul connection towards the AMF/MME in a PLMN. When the RAN node decides that it cannot serve the current PLMN any longer, it should indicate the AMF/MME to suspend the N2/S1-AP connection and store the context for resuming the N2/S1-AP connection in the future. Both the RAN node and the AMF/MME store the RAN node context information for resuming the N2/S1-AP connection. The RAN node and the 5GC should further adjust the heartbeat timer of the SCTP over N2/S1-AP interface to avoid heartbeat time out.

6.2.2	Procedures

Figure 6.2.2-1: Disconnecting a RAN node with the CN on the ground

1.	If the on-board RAN node is about to disconnect the feeder link towards the AMF/MME in the PLMN, the on-board RAN node should initiate the gNB/eNB SUSPEND procedure to inform the AMF/MME to store the context of the NG/S1 interface and update the heartbeat timer of the SCTP over N2/S1-AP interface. For the on-board RAN node that is about to leave, the on-board RAN may keep the related context as described in clause 9.2.6 of TS 38.413 [12] or clause 9.1.8 of TS 36.413 [14] unreleased and send the gNB/eNB SUSPEND REQUEST message including the following parameters:

a)	may include the suspend time or suspend period.

2	Upon receiving the gNB/eNB SUSPEND REQUEST message the AMF/MME:

a)	should store the context information as described in clause 9.2.6 of TS 38.413 [12] or clause 9.1.8 of TS 36.413 [14], necessary to resume the connection;

b)	should determine the heartbeat timer of the SCTP over N2/S1-AP interface based on configuration, e.g. ephemeris information of all on-board (ng-)eNB configured by the O&M or based on NWDAF inference and transfer the heartbeat timer to the lower layer;

c)	should send the gNB/eNB SUSPEND RESPONSEACKNOWLEDGE message with the heartbeat timer to acknowledge that it successfully updated the configuration data; and

d)	should stop sending paging requests to this RAN until the RAN resumes connection with AMF.

3	The RAN should adjust the value of heartbeat timer if received in step 2.

NOTE:	Whether the suspend/resume interaction between RAN and AMF/MME relies on an existing RAN procedure or new defined procedure depends on the RAN work group.

Figure 6.2.2-1: Reconnecting a RAN node with the CN on the ground

1.	If the on-board RAN node is reconnecting the feeder link towards the AMF/MME in the PLMN, the on-board RAN node should initiate the gNB/eNB RESUME procedure to inform the AMF/MME to resume the connection by sending the gNB/eNB RESUME REQUEST message includes the following parameters:

a)	should include the Global RAN Node ID;

b)	may include the resume time; and

c)	may include the context request indicator, if the on-board RAN does not store the AMF context.

2.	If the AMF/MME can active the on-board RAN context and resume the connection, the AMF shall update the heartbeat timer and send the gNB/eNB RESUME RESPONSE message with the updated heartbeat timer to acknowledge that it successfully resumed the connection and the corresponding context to RAN based on the context request indicator if received in step 1.

3	The RAN should update the value of heartbeat timer if received in step 2.

NOTE:	Whether the suspend/resume interaction between RAN and AMF/MME relies on an existing RAN procedure or new defined procedure depends on the RAN work group.

6.2.3	Impacts on services, entities and interfaces

AMF/MME:

-	Derive the heartbeat timer of the SCTP over N2/S1-AP interface based on configuration, e.g. ephemeris information of all on-board (ng-)eNB by O&M;

-	Support to suspend the TNL between the RAN node and the CN;

-	Support to suspend RAN context when the connection between RAN and AMF is interrupted

-	Support to resume the TNL between the RAN node and the CN;

-	Support to active RAN context when the connection between RAN and AMF is recovered

RAN:

-	Support for detecting the unavailability/availability of feeder link to AMF-MME and trigger N2/S1 suspend/resume procedures

-	Support to suspend the TNL between the RAN node and the CN;

-	Support to suspend AMF/MME context when the connection between RAN and AMF is interrupted

-	Support to resume the TNL between the RAN node and the CN;

-	Support to active AMF/MME context when the connection between RAN and AMF is recovered

NWDAF:

- Determination of the heartbeat timer of the SCTP over the N2/S1-AP interface for regenerative payload

******************** End of Changes ********************