3GPP TSG-SA/WG2 Meeting #161	S2-2402755

# Athens, GREECE, February 26 – March 01, 2024

Source:	CATT

Title:	KI#1, New solution – Support service exposure and interactions between 5GS and UTM for USS relocation

Document for:	Agreement

Agenda Item: 	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for Key Issue #1: Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions.

- Discussion

As described in the clause 5.1 of TR 23.700-59, the following aspects will be studied for KI#1:

-	whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting i.e.

-	Pre-mission flight planning and in-mission flight monitoring for UAVs.

-	C2 communication reliability.

-	The scenario of multiple USS serving different geographical areas corresponding to the UAV flight path.

NOTE:	In the scope of this key issue, UTM can represent any authorized aviation AF that may require interaction with the MNO for the functions listed above.

This paper is to discuss and support “The scenario of multiple USS serving different geographical areas corresponding to the UAV flight path”.

2	Proposal

It is proposed to agree the following texts in clause 6 of TR 23.700-59.

******* start of 1st change ********

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #y> |
| #x | x |  |
| #y |  |  |

6.X	Solution #X: Support service exposure and interactions between 5GS and UTM for USS relocation

6.X.1	Key Issue mapping

The following solution corresponds to the key issue #1 Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions as specified in clause 5.1.

6.X.2	Description

As described in clause 5.1, it is to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting the scenario of multiple USS serving different geographical areas corresponding to the UAV flight path. To support such scenario, the following aspects are included:

-	The procedure for UAV presence monitoring specified in clause 5.3.3 of TS 23.256 is reused, and the USS sets the service area(s) of the USS as the monitoring area.

-  Based on the UAV presence monitoring report, the source USS is relocated to the target USS.

-  The source USS initiates the UAV authorization revocation procedure.

-	The target USS initiates UAV Re-authentication procedure, and includes the source USS address, target USS address and relocation indication to be transparently delivered to the UAV.

6.x.3	Procedures

Figure 6.x.3-1: Support service exposure and interactions between 5GS and UTM for USS relocation

1.	The USS initiates the UAV presence monitoring, and the	UAS NF/NEF reports the UAV presence in the geographic area to USS based on the monitoring report from AMF or GMLC, as described in steps 1 to 5a of figure 5.3.3-1 of TS 23.256 [x].

In step 1, USS subscribes the events whether UAV is moving in or moving out of the monitoring area, and the USS sets the service area(s) of the USS as the monitoring area.

2.	The USS receives the UAV presence monitoring report and takes actions correspondingly.

2a. If the received report indicates that the target UAV moves out of the service area of USS (i.e. source USS), the USS may determine the target USS, e.g. based on the UE location in the report (if available). The source USS interacts with target USS, e.g. the source USS notifies the relocation to the target USS and provides the UAV related information (e.g. UAV context).

2b.	If the received report indicates that the target UAV moves into the service area of USS (i.e. target USS), the USS interacts with source USS, e.g. the target USS notifies the relocation to the source USS and retrieves the UAV related information (e.g. UAV context) from source USS.

NOTE: How the interaction between source USS and target USS is done is out of SA2 scope.

3.	The source USS initiates the UAV authorization revocation procedure as described in steps 1 to 5a of figure 5.3.3-1 of TS 23.256 [x].

4.	The target USS initiates UAV Re-authentication procedure as described in in steps 1 to 6 of figure 5.2.4.1-1 of TS 23.256 [x] with the following differences:

In step 1, the target USS may also include the source USS address, target USS address and relocation indication to be transparently delivered to the UAV.

6.x.4	Impacts on services, entities and interfaces

USS:

-	The USS sets the service area(s) of the USS as the monitoring area.

-  The source USS determines the target USS.

-  The target USS initiates UAV Re-authentication procedure, and includes the source USS address, target USS address and relocation indication to be transparently delivered to the UAV.

******* End of 1nd change ********