3GPP TSG|WG-2 Meeting #161 	S2-2402786

26 Feb. – 01 Mar. 2024, Athens, GREECE	(revision of S2-2400449)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | draft | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.501 draft CR for robust paging notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_Robust Notification_NR NTN |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Using satellite access typically has very limited power margin and usually requires LOS for best transmission performance. For Mobile Terminated service, the CM-IDLE may experience NLOS with poor reception channel conditions, which might miss important paging message (e.g. paging for emergency call). It is important to deliver paging message to those UEs under poor reception channel conditions, as this will greatly increase the success rate of MT service and improve user experience. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new feature to enable AMF to determine to apply robust paging notification to a UE if the paging to the UE fails. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Paging message for some important MT services might miss within NLOS coverage area . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1, 5.4.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.3	Paging strategy handling

5.4.3.1	General

Based on operator configuration, the 5GS supports the AMF and NG-RAN to apply different paging strategies for different types of traffic.

In the case of UE in CM-IDLE state, the AMF performs paging and determines the paging strategy based on e.g. local configuration, what NF triggered the paging and information available in the request that triggered the paging. If NWDAF is deployed, the AMF may also use analytics (i.e. statistics or predictions) on the UE's mobility as provided by NWDAF (see TS 23.288 [86]).

In the case of UE in CM-CONNECTED with RRC_INACTIVE state, the NG-RAN performs paging and determines the paging strategy based on e.g. local configuration, and information received from AMF as described in clause 5.4.6.3 and SMF as described in clause 5.4.3.2.

In the case of Network Triggered Service Request from SMF, the SMF determines the 5QI and ARP based on the downlink packet (if the SMF performs buffering) or the Downlink Data Report received from UPF (if the UPF performs buffering). The SMF includes the 5QI and ARP corresponding to the QoS Flow of the received downlink PDU in the request sent to the AMF. If the UE is in CM IDLE, the AMF uses e.g. the 5QI and ARP to derive different paging strategies as described in clause 4.2.3.3 of TS 23.502 [3].

NOTE:	The 5QI is used by AMF to determine suitable paging strategies.

Based on operator configuration, the 5GS supports the AMF and NG-RAN to apply robust paging notification alert for different UEs after a certain number of paging attempt fails.

# * * * * Next change (new)* * * *

5.4.3.X	Robust paging notification

Robust paging notification is an optional feature that allows the AMF to send robust paging notification request to NG-RAN in order to trigger the NG-RAN to allocate a dedicated Paging Alert Channel (PAC) to the UE after a certain number of paging fails. The PAC is used to notify UE to move to a location with better reception condition and respond to Paging Message.

The AMF determines to apply the robust paging notification for a service based on e.g. operator configuration, UE subscription. For example, if the paging for an emergency service fails, robust paging notification should be applied to the UE.

The time to trigger robust paging notification request by AMF is implementation depended and can be controlled by operator. For example, after a certain number of failed paging attempts, or after some time paging response still have not received by the AMF, robust paging notification can be triggered.

NOTE:	If the AMF determines to apply robust paging notification to a UE, Paging message can be sent in parallel with the robust paging notification alert by the AMF.

# * * * * End of changes * * * *