3GPP TSG-SA2 Meeting #161	S2-2402844

Athens, Greece, 26 Feb – 1 March 2024	(revision of S2-230xxxx)

Source:	NTT DOCOMO

Title:	KI#6: Sol 15 update

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract: the pCR proposes to update Solution#15

# 1. Introduction/Discussion

In Solution#15, the IMS session for standalone bootstrap data channel needs to be terminated into IMS AS. In the solution the Request URI contains a specific PSI, that needs to be identified by both DCSF and IMS AS. Based on the PSI, DCSF determines to instruct the IMS AS to assign resources for terminating media. IMS AS uses the PSI to determine that the SIP session is terminated in the IMS AS, e.g. to construct an SDP answer. More flexible approach would be to let the DCSF to control the IMS AS on SIP session termination via the Nimsas_SessionEventControl service. 
It is proposed to add a new service operation Nimsas_SessionEventControl_Create that can be used by DCSF to provide instructions to the IMS AS on how to process the SIP session.

In addition, the Editor's Note on how the PSI is configured to the UE is resolved.

It is also proposed that the UE may use the HTTP None-Match header when downloading the DC applications from DCSF to avoid downloading the data channel application if UE already stores the same version of the application that is available in the DCSF.

# 2. Text Proposal

It is proposed to capture following changes to TR 23.700-77.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[6]	3GPP TS 26.114: "Media handling and interaction".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TR 26.813: "Study of Avatars in Real-Time Communication Services".

[9]	3GPP TS 22.156: "Mobile Metaverse Services".

[10]	3GPP TS 24.229: "IP Multimedia Call Control based on SIP and SDP; Stage 3".

[11]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem".

[12]	3GPP TS 29.175: "IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application Server (AS) Services Stage 3 (Release 18)".

[13]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[14]	IETF RFC 3725: "Best Current Practices for Third Party Call Control (3pcc) in the Session Initiation Protocol (SIP)".

[15]	3GPP TS 24.341: "Support of SMS over IP networks".

[16]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[17]	ATIS-1000074: "Signature-based Handling of Asserted information using Tokens (SHAKEN)".

[18]	IETF draft-ietf-stir-passport-rcd-26: "PASSporT Extension for Rich Call Data".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[19]	IETF draft-ietf-sipcore-callinfo-rcd-08: "SIP Call-Info Parameters for Rich Call Data".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[x]	3GPP TS 24.275: "Management Object (MO) for Basic Communication Part (BCP) of IMS Multimedia Telephony (MMTEL) communication service".

# * * * * Next change * * * *

## 6.15	Solution #15: Solution to support standalone bootstrap data channel

### 6.15.1	Description

This solution addresses Key Issue #6 " Support of Standalone IMS Data Channel Sessions " and focuses on how to support establishing a (standalone) bootstrap data channel without accompanying application data channel.

When the UE initiates establishment of a standalone bootstrap data channel, the UE includes a specific PSI used for standalone bootstrap DC only in the INVITE request as the Request-URI to terminate the session and data channel towards IMS AS and DCSF.

The UE and network generally follow the existing procedures specified in current TS 23.228 [5] with some modifications, e.g. by only including SDP offer and answer for standalone bootstrap.

In a standalone IMS data channel scenario, the original UE typically communicates with the terminating UE using a specific DC application, such as Screen Share APP. To enable the user to decide which DC application to use, it would be better to display the list of available applications to the user, before initiating standalone IMS data channel for the actual DC application. To achieve this, the originating UE needs to establish a standalone bootstrap DC to download the DC application list before the user makes such a decision.

### 6.15.2	Procedures

When a UE initiates an IMS session with standalone bootstrap data channel, the procedure specified in clause AC.7.1 of TS 23.228 [5] applies with the following differences:

-	If the UE initiates a bootstrap data channel the SDP offer in step 1 only includes media component for originating bootstrap data channel;

-	The procedure terminates on originating network.

-	The UE may release the IMS session after the DC application(s) have been downloaded from the DCSF.

Figure 6.15.2-1depicts a signalling flow diagram for establishing a standalone bootstrap data channel.

Figure 6.15.2-1: Standalone Bootstrap Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

1-9.	Step 1-9 from clause AC.7.1: (Bootstrap Data Channel Setup Signalling procedure) in TS 23.228 [5] is performed with the differences that:

In step 1, The UE#1 sends the SIP INVITE request, the Request URI of the SIP INVITE message is a specific target URI used for standalone bootstrap DC only. The initial SDP contains only bootstrap data channel offers for UE#1.

NOTE 1:	UE#1 can perform step 1 immediately after completing the SIP registration procedure.

NOTE: the target URI can be configured to the UE e.g. via OMA device management as specified in 3GPP TS 24.275 [x].

In step 5, DCSF determines that UE#1 requests to establish a standalone bootstrap DC based on the value of the Called ID and creates only the originating side media information.

In step 6, DCSF instructs the IMS AS how to set up bootstrap data channel with MF for originating side only.

In step 8, IMS AS instructs MF to allocate required data channel media resources for originating side only. IMS AS request creation of only one Media Termination which representing the local bootstrap media to be terminated.

In step 9, IMS AS responds to the MediaInstruction request received in step 6.

10.	DCSF initiates a Nimsas_SessionEventControl_Create request to the IMS AS. The request contains instruction to terminate the IMS session in the IMS AS.

11. IMS AS responds with Nimsas_SessionEventControl_Create response.

12. The DCSF responds to the Notify Request received in step 3.

13.	IMS AS terminates the IMS session based on the instruction received in step 10, and sends 18X response with the SDP answer through S-CSCF and P-CSCF to UE#1.

14-15.	The session with bootstrap data channel is established.

16.	The bootstrap data channels have been established between originating MF or MRF and UE#1. The UE#1 downloads the data channel applications by sending HTTP request messages to MF or MRF tvia the established bootstrap data channel with its data channel capabilities. The UE may include If-None-Match header into the HTTP request to avoid downloading the data channel application if UE already stores the same version of the application that is available in the DCSF.

The MF or MRF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 based on their data channel capabilities and their choices through MF or MRF.

17-18.	If the established bootstrap DC is not used for a period of time, UE#1 sends SIP BYE to release the bootstrap DC to save power.

### 6.15.3	Impacts on Existing Nodes and Functionality

This solution has some requirements on the existing IMS nodes below:

DCSF:

-	Determines that the UE requests to establish a standalone bootstrap DC based on the called ID.

-	Creates only the originating side media information.

IMS AS:

-	Terminates the IMS session based on the instruction from DCSF.

-	New service operations Nimsas_SessionEventControl_Create can be used by DCSF to provide instructions to the IMS AS on how to process the IMS session, e.g. to terminate the IMS session to the IMS AS. If no instructions are received, the IMS AS proceeds as specified in Rel-18 TS 23.228.

Editor's note:	 Further impacts are FFS.

# * * * * End of changes * * * *