SA WG2 Meeting #161	S2-2403179

Athens, Greece, February 26 – March 01, 2024	(revision of S2-2402405)

Source:	LG Electronics

Title:	KI#1, KI#2 New Sol: Identifying the Human User of UE based on User Identity Profile stored in the UDM

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This paper proposes a new solution for KI#1 and KI#2.

Discussion

This paper proposes a new solution for KI#1 and KI#2.

Proposal

It is proposed to agree the following changes into TR 23.700-32.

* * * * Start of 1st Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> |
| #1 |  |  |  |  |
| #2 |  |  |  |  |
| #X | X | X |  |  |

* * * * Start of 2nd Change * * * *

!!! All New Texts !!!

6.X	Solution #X: Identifying the Human User of UE based on User Identity Profile stored in the UDM

6.X.1	Key Issue mapping

This solution addresses Key Issue #1 and Key Issue #2.

6.X.2	Description

In this solution, it is assumed that a User Identifier is unique at least within the operator's network.

The User Identity Profile contains following information:

-	A User Identifier

-	credentials for authentication of the User Identifier

-	Specific service settings and parameters

-	Linked 3GPP subscription (SUPI)

Editor's note:	It is FFS whether credentials are stored in the User Identity Profile. It depends on authentication procedure which will be determined by SA3.

Editor's note:	Details of Specific service settings and parameters are FFS.

The User Identity Profile is provided to the 5GC by using External Parameter Provisioning procedure described in clause 4.15.6.2 of TS 23.502 [x]. When the AF provides the U between ser Identity Profile, GPSI of subscription linked with the User Identifier is provided so that 5GC aware of Link 3GPP subscription and User Identifier.

When a user intends to use a UE using the User Identifier, the UE triggers Registration procedure by including User Identifier of the user. The AMF triggers authentication of the User Identifier and, if authentication is successful, the AMF stores the User Identifier and registers to the UDM of the User Identifier. The AMF provides Registration Accept to the UE with indication that service for the User Identifier is allowed. The User Identifier is transferred to the SMF/PCF of existing PDU Session or new PDU Session. If the SMF/PCF receives the User Identifier, the SMF/PCF gets the User Identity Profile information from the UDM/UDR and can apply the User Identifier specific policies.

6.X.3	Procedures

6.X.3.1	Procedures for provisioning of User Identity Profile

Figure 6.X.3.1-1: Provisioning of User Identity Profile

1.	If the AF wants to create/update/delete User Identity Profile, the AF provides User Identity Profile together with GPSI of linked 3GPP subscription by triggering Nnef_PrameterProvision_Create/Update/Delete service operation.

2.	If the AF is authorized by the NEF, the NEF selects UDM by using the GPSI of linked 3GPP subscription. The NEF triggers Nudm_PrameterProvision_Create/Update/Delete service operation.

3.	If the AF is authorised by the UDM to provision the parameters for the subscriber (represented by GPSI of linked 3GPP subscription), the UDM requests the UDR to create, update or delete the "specific service settings and parameters" in the User Identity Profile via Nudr_DM_Create/Update/Delete Request. The stored information can be used by the PCF to apply User Identifier specific policies.

4.	UDR responds the request with Nudr_DM_Create/Update/Delete Response.

5.	UDM responds the request with Nudm_ParameterProvision_Create/Update/Delete Response.

6.	NEF responds the request with Nnef_ParameterProvision_Create/Update/Delete Response.

6.X.3.2	Procedures for Activation of User Identifier

Figure 6.X.3.2-1: Activation of User Identifier

1.	The UE is registered and established PDU Sessions without User Identifier. The procedure follows existing procedures.

2.	The UE sends NAS message e.g. Registration Request by including User Identifier of the user.

NOTE:	If Registration Request message is used and there is no NAS security (e.g. if step 1 is not performed), the User Identifier can be transmitted after NAS security is established e.g. by using non-clear text IEs.

3.	Based on the User Identifier provided by the UE, AMF triggers authentication of the User Identifier.

NOTE:	If the UE is not registered to the network (e.g. if step 1 is not performed), the AMF first triggers primary authentication of the UE using existing procedure. After the primary authentication of the UE is completed, the AMF triggers authentication of the User Identifier.

4.	The AMF triggers Nudm_UECM_Update service to notify the UDM that the User Identifier is activated and the User Identifier is served by the UE.

NOTE:	If the AMF does not performed registration to the UDM, (e.g. if step 1 is not performed), the AMF can trigger Nudm_UECM_Registration service to register the AMF and notify that the User Identifier is activated.

The UDM may check whether the User Identifier is used by other UE (i.e. check whether the User Identifier is already activated by other UE). If the User Identifier is used by other UE, based on local policy, the UDM either rejects the registration or  sends Nudm_UECM_DeregistrationNotification to the other UE which is using the User Identifier to deactivate User Identifier over the UE.

5.	The AMF retrieves User Identity Profile from the UDM by triggering Nudm_SDM_Get by providing User Identifier. Based on the received information, the AMF may check whether the UE is allowed to be used by the User Identifier.

6	If the User Identifier is allowed to get a service with the UE, the AMF sends NAS message e.g. Registration Accept to the UE with indication that the User Identifier is allowed. The AMF stores the User Identifier in the UE Context. Otherwise, the AMF notify the UDM that the User Identifier is deactivated and sends NAS message e.g. Registration Accept to the UE with indication that the User Identifier is not allowed and procedure stops.

7.	The AMF triggers Nsmf_PDUSession_UpdateSMContext to release existing PDU Session(s).

8.	The UE sends PDU Session Establishment Request message.

9.	The User Identify Profile received in step 5 may contain DNNs and S-NSSAIs that the user is permitted to access. Based on the information User Identity Profile information and the slices that the subscription is allowed to access, the AMF determines whether to proceed PDU Session Establishment procedure. If the AMF determines to proceed, the AMF triggers Nsmf_PDUSession_CreateSMContext and includes the User Identifier.

NOTE:	The DNN and S-NSSAIs that the user is permitted does not override UE's subscription, i.e. DNN and S-NSSAIs that are not available to the UE is not allowed to the user.

10.	The SMF retrieves User Identity Profile from the UDM by triggering Nudm_SDM_Get by providing User Identifier. Based on the received information, the SMF may update QoS based on operator policy.

11.	The SMF may create SM Policy Association with the PCF and includes User Identifier. Based on the received User Identifier, the PCF can retrieve the User Identity Profile from the UDR to apply User Identifier specific policy.

12.	The SMF registers the PDU Session to notify the UDM that the User Identifier is using the PDU Session.

13. The SMF provides PDU Session Establishment Accept message to the UE.

6.X.4	Impacts on services, entities and interfaces

UE:

-	support handling of User Identifier during the Registration procedure.

AMF:

-	support receiving User Identifier from the UE and triggers authentication.

-	support registering the activated User Identifier.

SMF:

-	support receiving User Identifier from the AMF and registering the PDU Session.

PCF:

-	support receiving User Identifier from the SMF and retrieves User Identity Profile from UDR.

UDM:

-	support storing of User Identity Profile.

-	enforce that User Identifier does not activated simultaneously over different UEs.

NEF, UDM, UDR:

-	support provisioning of User Identity Profile.

* * * * End of Changes * * * *