SA WG2 Meeting #161	S2-2403453

February 26 – March 1, 2024, Athens, Greece

Source:	InterDigital Inc., Samsung

Title:	Key Issue #2, New Solution: AF Session with Energy savings criteria

Document for:	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper proposes solution for Key Issue #2.

# 1	Discussion

This paper proposes a new solution for Key Issue #2 - Subscription and policy control to support energy efficiency and energy saving as service criteria.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-66 V0.3.0.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| 1 | x |  |  |  |
| 2 | x |  |  |  |
| 3 | x |  | x |  |
| 4 | x |  |  |  |
| 5 | x |  | x |  |
| 6 | x |  |  |  |
| 7 |  | x |  |  |
| 8 |  | x |  |  |
| 9 |  | x |  |  |
| 10 |  | x |  |  |
| 11 |  | x | x |  |
| 12 |  |  | x |  |
| 13 |  |  | x |  |
| 14 |  |  | x |  |
| 15 |  | x | x |  |
| Y |  | x |  |  |

* * * * Next Change – All New Text * * * *

## 6.y	Solution #y: AF Session with Energy Savings Criteria

### 6.y.1	Key Issue mapping

The solution applies to Key Issue #2 "Subscription and policy control to support energy efficiency and energy saving as service criteria".

### 6.y.2	Functional Description

The procedure for setting up an AF session with required QoS TS 23.502 [x] clause 4.15.6.6, allows an AF to request to reserve resources for a session by invoking Nnef_AFSessionWithQoS_Create, including the QoS requirements. The PCF derives the PCC rules based on the QoS parameters received from the NEF in a Npcf_PolicyAuthorization_Update request message. The PCF sends the SMF updated policy information about the associated PDU Session as required.

In this solution it is proposed to enhance the procedure for setting up an AF with required QoS, so that the AF can provide energy savings requirements in an energy saving profile that includes the desired energy savings (or consumption) level, the energy credit, required energy efficiency and communication service features (e.g UP security) that can be adjusted (e.g., optimized, turned off) according to required energy savings level, the configured energy credit, or the required energy efficiency. Examples of 5GS communication service features that can be subject to energy saving adjustments are User Plane (UP) security, Reflective QoS etc. The PCF determines the PCC rules for the session based on the energy savings profile and energy consumption analytics data from an NWDAF or an EECF (e.g. the EECF of solution #10) and updates the SMF for the PDU Session accordingly. The mechanism provides means for energy savings policy control on PDU Session features (e.g., UP Security) to enable the network to have the flexibility to determine acceptable trade-offs between the service performance/reliability/security and the service energy consumption level required and apply policy control accordingly.

Editor’s Note:  The relationship between energy saving and communication features is FFS

### 6.y.3	Procedures

Figure 6.y.3-1 below shows an AF session with required QoS (TS 23.502 [x] clause 4.15.6.6)with enhancements to support energy savings criteria for the session. The changes that are proposed by this solution are described below.

Figure 6.y.3-1: Setting up an AF session with required Energy savings procedure

1. The AF sends a request to reserve resources for an AF session with energy savings criteria using Nnef_AFsessionWithQoS_Create request message including QoS parameters and energy saving profle information which may include energy savings level, the energy credit, required energy efficiency and optionally features subject to energy savings control/adjustement (e.g UP security).

2. The NEF checks authorization for the AF request and may apply policies to verify that that feature based energy savings control can can be authorized for the AF and the features allowed to be under energy related requirement, e.g. savings control., the energy credit, required energy efficiency If the authorization is not granted, the NEF rejects the request otherwise proceeds with the following steps.

3. The NEF forwards received parameters to the PCF in the Npcf_PolicyAuthorization_Create request.

4. The PCF sends to the NWDAF or EECF a request to retrieve energy consumption analytics by providing Application Identifier, QoS parameters and the list of features subject to energy control/adjustment (e.g., which can be adjusted or enabled/disabled for the communication service).

The PCF receives the energy consumption data from the NWDAF/EECF including per feature relative contribution to the energy consumption associated with the service/session.

5. The PCF determines a policy configuration for the PDU Session (i.e. PCC rules, UP Security Policy, etc.) using the energy consumption data and requested session energy related profile to achieve the desired energy requirements in the AF request (e.g. the energy saving level, the energy credit, required energy efficiency). The PCF determines the features for which PCC rules and other policy configuration information that needs to be established or updated for the adjustment/disablement of the features accordingly.

6. The PCF sends Npcf_PolicyAuthorization_Create response informing the NEF/AF whether the request is granted and includes information on the adjusted or de/activated features and  energy policy and requirements for the session.

7. The PCF updates the SMF with the corresponding policy indicating rules for the features adjustment or de/activation for energy related requirements, .e.g. energy savings, energy credit, required energy efficiency etc.. The SMF applies the policy with energy savings rules to the PDU Session and notifies the PCF of success or failure.

8. The PCF notifies the NEF/AF the SMF when the establishment of the transmission resources with corresponding energy requirements succeeded or failed.

### 6.y.4	Impacts on existing services, entities and interfaces

AF:

- Sends: energy savings profile (energy savings level, list of features subject to potential energy savings control/adjustments)

- Receives: energy savings level, the energy credit, required energy efficiency, list of associated adjusted features.

NEF:

- Receives from AF and sends to PCF: energy savings profile.

- Receives from PCF and sends to AF: energy savings level, the energy credit, required energy efficiency, list of associated adjusted features.

PCF:

- Receives from NEF: energy savings profile.

- Sends QoS parameters and list of adjustable features to the EECF.

- Receives total energy consumption cost for the session and individual feature relative contribution to the total energy consumption cost.

- Sends policy configurations and PCC rules derived for energy requirements for the session to the SMF.

- Sends to NEF: energy savings level, the energy credit, required energy efficiency, list of associated adjusted features.

EECF:

- Receives from PCF: QoS parameters and list of features subject to energy savings for the session.

- Sends to PCF: energy related parameters, e.g. energy consumption cost (total, per feature), energy consumption within a time window.

* * * * End of Changes * * * *