3GPP SA WG2 Meeting #161	S2-2403738

26 February – 01 March, 2024, Athens, Greece                                 (rev of S2-2403440)

Source:	NEC

Title:	KI#1, New Sol: UPF selection based on running functionalities

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2/ Rel-19

Abstract of the contribution: The contribution proposes a solution to Key Issue #1: Selection of UPF providing a selected user plane functionality in TR 23.700-63.

1. Introduction

This contribution proposes a solution to Key Issue #1: Selection of UPF providing a selected user plane functionality. It focuses on extensions or enhancements on UPF advertising capabilities and selection of UPF supporting specific user plane functionalities.

-	Whether there is a need and how to extend the existing UPF advertising capabilities.

-	Whether and how the input parameter(s) for UPF selection by SMF should be enhanced to support selection of UPF supporting specific user plane functionalities, e.g. NAT, Packet Inspection, etc.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-63.

* * * Start of 1st change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #X | X |  |

* * * End of 1st change * * * *

* * * Start of 2nd change * * * *

## 6.X	Solution #X: UPF selection based on the status of the supported functionalities

### 6.X.1	Key Issue mapping

The solution applies to Key Issue #1: Selection of UPF providing a selected user plane functionality.

### 6.X.2	Description

As described in the Key Issue #1, UPF advertises the features it supports in the NF profile of UPF registered in NRF. A UPF implementation may support more optional functionalities like NAT, Packet Inspection, etc. An SMF leverages the exposed UPF information for UPF selection.

In the context of TS23.501[2], the exposed features are introduced as following parameter(s) and information that may be considered in the UPF selection by the SMF.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

Note:

-	The capability or functionality to be provided may be further described as a part of UPF Provisioning Information in the NRF.

In UPF implementations, individual functionality can be enabled/ disabled after the provisioning. In such a case, the individual functionality may be seen as to be provided from the NF profile perspectibe because it has been provisioned. But it may not be available.

The objective of the UPF selection in the SMF is to discover available UPF instance(s). NFDiscovery can be used to clarify if individual functionalities are supported or non-supported.

This solution further clarifies if the individual functionalities are available or unavailable out of the discovered UPF(s) so that SMF can acculately select available UPF(s)

Assumptions:

- As described in TS23.501 [2], OAM configures NF Profile for the UPF.

### 6.X.3	Procedures

Figure 6.X.3-1 Procedure to notify status of functionalities in UPF

- UPF invokes Nnrf_NFManagement_NFRegister service to request NRF to register the configured NF Profile for the UPF.

- In PDU session establishment, for example, SMF invokes Nnrf_NFDiscovery service to request NRF to discover UPF(s) that supports event exposure service to notify status of running functionalities (e.g. active, inactive of NAT, Packet Inspection etc).

- NRF responds SMF to give NF Profile for the UPF that matches the request.

- SMF has:

- a list of UPF(s) that supports targeted functionalities

- UPF informs status of the individual functionalities in the discovered UPF(s)

- NRF informs status of the individual functionalities in the discovered UPF(s)

Editor’s note1: Which operation  is used and impacts are for futher study.

- Leveraging Nnrf_NFManagement service operations

- N4 Node Level Procedures

Editor’s note2:  Impacts to the existing PDU session needs to be studied.

- SMF executes UPF selection based on the status of the supported functionalities in the UPF. For example, UPF in which the status of NAT is active is selected if UPF with NAT is requested.

Editor’s Note:  how UPF know which SMF is to be notified and how to notify it as no SMF endpoint information.

Editor’s Note:   It is FFS for SMF what is the difference of the one functionalities in the UPF is disable comparing to the one functionalities is not included in the UPF NF profile.

### 6.X.4	Impacts on services, entities and interfaces

SMF:

- UPF selection based on the status of the supported functionalities

UPF:

- Notification about status of supported functionalities

NRF:

- Notificationy about status of supportedrunning functionalities

* * * End of 2nd change * * * *