3GPP SA WG2 Meeting #S2-161	S2-2403833

26 February - 01 March, 2024, Athens, Greece

Source:	MediaTek Inc.

Title:	Identification and association for two subscriptions/SUPIs of a DualSteer Device

Document for:	Agreement

Agenda Item:	19.13

Work Item / Release:	FS_MASSS/Rel-19

Abstract of the contribution: This paper proposes a solution to resolve the 5GS identifies and associates the two subscriptions/SUPIs for DualSteer

Discussion

In this paper, it proposes a solution to resolve the following Key issue:

"-	Whether and how the 5G System identifies and associates the two subscriptions/SUPIs for DualSteer."

Proposal

It proposes to include the below solution to TR 23.700-54

*** Start of changes ***

6.0	Mapping of solutions to Key Issues

6.x	Solution #X: Identification and associations for two subscriptions/SUPIs of a DualSteer Device

6.X.1	Key issue mapping

In this solution, it addresses the following issue:

-	Whether and how the 5G System identifies and associates the two subscriptions/SUPIs for DualSteer.

6.X.2	Functional Description

A DualSteer device has two subscriptions/SUPIs, sharing one subscription profile from the same operator. However, the 5GC does not know whether these two subscriptions/SUPIs are in the same DualSteer device. Therefore, the DualSteer device and the 5GC which supports the DualSteer cannot use the DualSteer feature for traffic transmission. To resolve the above issue, the UE(s) needs to indicate to the 5GC that the two subscriptions/SUPIs are in the same DualSteer device.

To achieve this, the following principles are applied:

-	Two subscriptions/SUPIs require separate registration procedures for each of the subscriptions/SUPIs in the DualSteer device (regardless of non-simultaneous transmission or simultaneous transmission)

-	The UE is aware of belonging to the DualSteer device to support DualSteer feature.

NOTE 1: The UE can be (pre)-configured know which subscriptions/SUPIs are used for DualSteer feature

For Registration below cases are considered and apply to DualSteer device with two subscriptions/SUPIs for non-simultaneous transmission with a single UE and for simultaneous transmission with separate UEs:

Registering to same PLMN:

In case of single UE , UE is aware of SUPIs in the DualSteer device, when the UE triggers the registration procedure for one of the subscriptions/SUPIs (e.g., SUPI1), the UE includes SUCI1 of SUPI1 along with the SUCI2 of SUPI2 in the Registration Request to the 5GC. When the 5GC receives the information of SUCI1 of SUPI1 along with the SUCI2 of SUPI2, the 5GC knows the UE supports DualSteer feature for the SUPI1. If the 5GC does not support DualSteer feature, the 5GC ignores the SUCI2 of SUPI2 and considers the registration as normal registration for SUPI1.

Similarly to the registration of SUPI1, when the UE triggers the registration procedure for the SUPI2, the UE includes the SUCI2 of SUPI2 along with the SUCI1 of SUPI1 in the Registration Request to the 5GC. When the 5GC receives the information of SUCI2 of SUPI2 along with the SUCI1 of SUPI1, the 5GC knows the UE supports DualSteer feature for the SUPI2. If the 5GC does not support DualSteer feature, the 5GC ignores the SUCI1 of SUPI1 and considers the registration as normal registration for SUPI2.

When the 5GC receives the above registration information from SUPI1 and SUPI2, the 5GC knows SUPI1 and SUPI2 are associated to the same DualSteer device. Therefore, the 5GC and the single UE in the DualSteer Device can use the DualSteer feature (e.g., DualSteer switching) in the serving PLMN.

If the serving PLMN is not the subscriptions owner of the SUPI1 and SUPI2, the serving PLMN needs to forward the received information from SUPI 1 and SUPI 2. Otherwise, the DualSteer feature cannot work since the subscription owner does not know it needs to perform the DualSteer for the DualSteer device.

Registering to different PLMNs:

Similarly, if the single UE triggers the separate registration procedures for SUPI and SUPI2 to different PLMN (e.g., PLMN1 and PLMN2), the similar behavior of the UE as registering to the same PLMN..

If one of the PLMNs are the subscription owner of SUPI1 and SUPI2 (e.g., PLMN1), the other PLMN (e.g., PLMN2) needs to forward the received information from SUPI2 to the subscription owners (e.g., PLMN 1) for using DualSteer feature.

If none of PLMN1 and PLMN2 are the subscription owner of SUPI1 and SUPI2, PLMN1 and PLMN2 need to forward the received information from SUPI1 and SUPI 2 to the subscription owner for using DualSteer feature.

6.X.3	Impacts on existing services, entities and interfaces

UE with two SUPIs (SUPI1 and SUPI2) in DualSteer Device:

- Needs to include SUCI1(2) along with SUCI2(1) in Registration Requests

- Aware of belonging to DualSteer Device/ support for two SUPIs

Two separate UEs in DualSteer Device:

-	Aware of belonging to DualSteer Device

-	Support implementation-specific inter-UE identifier sharing

5GC:

-	Aware of whether supporting dual steer feature or not

V-AMF:

- Needs to forward the pair information of SUCI1(2) along with SUCI2(1) to the UDM of subscription owner of SUPI1 and SUPI2

H-UDM:

- Needs to access the sharing subscription profile to determine whether two subscriptions/SUPIs are for DualSteer

- Needs to determine whether two subscriptions/SUPIs are in the same DualSteer device and to decide whether to perform DualSteer features for the DualSteer device

*** End of changes ***