3GPP TSG-SA2 Meeting #161	S2-2402820

Athens, Greece, February 26 – March 3, 2024	(revision of S2-xxxxxx)

# Source:	Siemens AG, Nokia, Nokia Shanghai Bell

# Title:	New WID on Source Traffic Descriptors for URSP

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on Source Traffic Descriptors for URSP

Acronym: TEI19_URSP_STD

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a Feature

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

# 3	Justification

One of the emerging roles of 5G UE is to act as a gateway that provides connectivity to applications of devices in a capillary network “behind” the UE. An example are devices in an AGV that connect to the UE via an on-board wired Ethernet Network. When the UE determines the PDU session for an application of a specific device “behind the UE”, current traffic descriptor information is not sufficient.

Considering that the traffic-originating application can be anywhere in the capillary network “behind” the UE, the source IP or other source information may be used to differentiate the application traffic from the capillary network to the UE. The current traffic descriptors only match the destination address and/or the destination port.

The IP address range, MAC address range and port number range are also useful to differentiate application traffic running over a certain range of addresses and ports. For example, factory automation and process control, the all capillary devices communication via real-time Ethernet protocols could be mapped onto a particular address range.

# 4	Objective

The objective of this work item:

WT-1: Ability of UE to determine to route outgoing traffic for its connected devices based on source information in URSP.

## TU estimates and dependencies

| Work Tas ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-1 |  | 0.5 | No | None |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.503 | Support source address and related information in URSP in Table Table 6.6.2.1-2 | TSG#103 |  |

# 6	Work item Rapporteur(s)

Ling Zhang, Siemens AG (zhangling@siemens.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

Supporting IM name

Siemens AG

Nokia

Nokia Shanghai Bell

Philips International B.V.

CableLabs

Charter Communications, Inc