SA WG2 Meeting #S2-161	S2-2403151

26 February - 3 March, 2024, Athens, Greece

- Title:	TR 23.700-63, New solution for KI#3: provisioning of information for handling of headers in UPF

- Source:	Vodafone, Nokia, Nokia Shanghai Bell, Deutsche Telekom, ZTE

- Document for:	Approval

Agenda Item:

Abstract:	This pCR proposes a solution for KI#3

# 1	Background

TS 23.501, in clause 5.8.5.6, specifies how SMF can provide a container for header enrichment as part of the FAR to the UPF. SMF can as well provide metadata to UPF for this to add the metadata to the traffic over SFC.

However, the viability of using all these possibilities in connection with a business contract, with Application providers, is quite reduced when these mechanisms have to be statically configured in one of the network nodes.

In addition, the arrival of different transport protocols and openness of the network creates the need to evolve the mechanisms as standardised in 3GPP. Different solutions are explored in IETF, but do not consider the trend of programmable networks.

The proposal in this tdoc leverages on the SFC solution with the difference of handling in uplink and downlink direction, including the detection. It also expands the current functionality of header enrichment to enable the request from an operator platform.

# 2	Proposal

It is proposed to agree the text below for inclusion into TR 23.700-63.

* * * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 |
| Solution X: Provisioning of information for header handling |  |  | X |
| #2 |  |  |  |

NEXT CHANGE

## 6.X	Solution #X: Provisioning of information for header handling

### 6.X.1	Description

Network operators can reach agreements with Application Service Providers to handle traffic specific to their applications in a particular way and/or under specific conditions, in terms of SLAs between both business parties.

Automation capabilities of 5G for network slicing, charging, interactions with AFs, use of analytics, etc permit MNOs to engage in agreements with Application service providers in a dynamic way compared to previous technologies and to provide relevant configurations for devices and network nodes using rules and policies to support such agreements.

One way of implementing agreements with an ASP is by exchanging in-band information included in the relevant protocols. Client, server or proxies in the traffic path can insert specific information or use specific protocols that imply a particular behaviour for the receiving entity. An example of insertion of information in Release 18 is the use of ECN for L4S to permit rate adaptation for a particular flow.

Release 18 specifications have also included functionality for Service Function Chaining capabilities so that an Application Function can request a particular steering of user plan traffic towards specific service functions in the N6-LAN. Additionally, in earlier releases, the enrichment of packet headers was possible in the uplink direction by including specifically the IMEI as part of the header. This capability has evolved in 5G to be more generic and make use of containers in FAR rules, however, it is still not available for request from the AF/operator platform and no functionality exists for notification upon detection of headers.

Recenly, different standards organizations are engaged in developing protocols and market is developing mechanisms that permit the network to recognise the content so specific handling can be provided, while keeping the content encrypted.

The solution leverages existing functionality to address mechanisms that facilitate SLAs, in an environment where the variety of mechanisms and protocols keep on growing, using in-band information specific to the particular agreement:

- Header/tag(s) handling request in N6 or in N3 and reporting.

Different use cases may require procedures at different levels. The HTTP traffic is being reduced while encrypted protocols grow, so any potential insertion or detection would need to be at different layers, mainly transport and IP. To make the mechanism generic for different cases, it is proposed to use a header Tag Id (hTId) to identify a rule referred to by an AF.

### 6.X.2	Procedures

#### 6.X.2.1	Insertion/detection request

The proposed rule contents shall include:

-	Conditions for header/tag handling e.g. always, at initiation, during certain period;

-	Actions: Add, Remove, Replace, Modify, Detect one or more header/tag(s);

-	Reporting conditions: Threshold based, periodical with the relevant threshold values and periodicity;

-	Reporting content e.g. header/tag “x” detected, header/tag “a” is replaced by header/tag “b”, header/tag “c” is added, etc.

- 	Information relevant to parties in SLA

Leveraging on the NF service Nnef_TrafficInfluence, and following the description in clauses 5.6.7 and 5.6.16 of TS 23.501, an AF may request the SMF, via NEF and PCF, handling some header/tag to the protocol at the user plane for:

- a particular S-NSSAI/DNN and

- a particular application and/or

- a particular UE/group of UEs or any UE.

The actual rule(s) is(are) provided as a container/metadata in the relevat Nnef_TrafficInfluence operation (create/update/delete) by the AF, together with the direction for handling, i.e. uplink, downlink and the identifier of the rule. As opposed to influencing on routing or SFC, in this case the AF request may not always need to be linked to an ongoing PDU session. The request can be applied to the ongoing or future PDU session.

In order to handle traffic for a particular application, the procedure for AF traffic influence would need to be complemented with the use of Nnef_PFDManagement (create/update), in push or pull mode, that the AF would have created prior to the influencing request.

The following sequence is extracted from TS 23.502 clause 4.3.6.2 to apply for this solution:

In step#1, the AF inserts the rule identifier and the container. In step#2, NEF will store/update the received data in the UDR for data set pointing to application data.

At notification from UDR, PCF validates whether the header/tag rule identifier corresponds to an authorized AF based on local configuration. If the validation has succeeded the PCF includes the rule identifier and the rule metadata, if received, in the PCC rule. If the AF requests reporting on header/tag detection, the PCF includes in the PCC rule(s) the information required for reporting the event.

In step#5, the PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new policy information about the PDU Session.

In step#6, SMF may need to associate the policy with a PFD (already pushed or by pulling from PFDF(NEF)) before installing PDR/FAR and URR (if reporting required) rules into the UPF.

Editor’s Note:  Whether header handling exposure is part of Nnef_TrafficInfluence or another service (e.g. a new service) is FFS

#### 6.X.2.2	Header/tag reporting/notification leveraging Nnef_TrafficInfluence_Notify

When a detection action is requested by the AF, SMF and UPF are instructed to report information included by the client or server side under the SLA. The instruction for detecting a header/tag may or may not be connected with an insertion/removal/modification/replacement.

In case the detection action is not connected with any insertion/removal/modification/replacement, the procedure in TS 23.502 clause 4.4.2.2 is applicable to this solution upon detection in the UPF of a PDR associated to a URR.

Upon detection of the header/tag requested by the AF, the N4 Session Report with “Start of Traffic (usage report)” is assumed to be used by UPF. This usage report shall include the information related to the header/tag that has been detected.

If the detection action is associated with an insertion/removal, modification or replacement, the N4 Session report with “Start of Traffic (usage report)” is proposed to be used as well by the UPF. The usage report shall include instead information about the header/tag detected and the action performed on it.

The following sequence from TS 23.502 clauses 4.3.6.3 to apply for this solution upon notification from the UPF:

After the step 2b, the actions the AF may execute are not depicted; those will depend on the agreement with the MNO.

NOTE:      Reporting detection of a header for Any UE or for some popular applications can cause UPF to issue lots of reports and consequently high signaling load. Requesting reporting of a header/tag detection in UEs traffic needs to be considered with care.

### 6.X.3	Impacts on services, entities and interfaces

- NEF

  - Nnef_TrafficInfluence service, adding optional header tag rule Id, container/metadata, direction (uplink/downlink) and handling new event in the Event ID reported to AF

- UDR

  - Nudr_DataManagement service, adding new data subset in the Application data set.

- PCF

  - PCC rule to add a header tag identifier and metadata

- SMF

  - Nsmf_EventExposure service, to include new event for notification to the AF

  - To depict PDR/FAR/URR rules handling for header/tag handling.

  - N4 updates

- UPF

  - N4 updates

  - Header handling capability, for insertion and detection