SA WG2 Meeting #163	S2-2405993

# 27 – 31 May 2024, Jeju, South Korea

Source: 	NTT DOCOMO

Title: 	KI#1, Interim conclusion

Document for: 	Approval

Agenda Item: 	19.15 (FS_AIML_CN)

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion for KI#1.

# 1	Discussion

Based on the solutions documented in TR 23.700-84, this paper proposes a set of principles to be considered in the normative work for KI#1.

# 2 Proposal

It is proposed to update TR 23.700-84 according to the following text.

********** First Change (All new text) **********

# 8	Conclusions

## 8.X	Interim Conclusions for Key Issue #1: Enhancements to LCS to support Direct AI/ML based Positioning

It is agreed to consider the following principles for the normative work:

- For LMF-side Direct AI/ML positioning of UE, LMF shall provide the following functionalities:

- LMF determines the location/position of UE directly using an ML model.

- LMF monitors the accuracy of Direct AI/ML UE positioning which may trigger ML model retraining in the training entity.

- LMF may collocated with AnLF.

Editor's Note: Whether multiple ML models are needed for positioning UE in different scenarios/areas/times or not is FFS.

- The following options may be used for training LMF-side ML model for Direct AI/ML positioning of UE:

- LMF trains the ML model which is used locally inside the LMF; this option is preferred if all the required training data are available in the LMF.

- MTLF trains the ML model which is transferred (using the existing service operations) to the LMF (collocated with AnLF); this option is preferred if training data from multiple source NFs are needed.

- OAM trains the ML model and configures it in the LMF, this option is preferred if the operator may provide the customized ML model.

- The data for model training, inference and model performance monitoring for Direct AI/ML based positioning will be decided by RAN WGs. SA WG2 will align with RAN WGs.

Editor's Note: Whether additional data (and its corresponding sources) beyond that defined by the RAN WGs is needed or not is FFS.

********** End of Changes **********