3GPP TSG-SA WG2#163	S2-2406031

Jeju, South Korea, 27 – 31 may 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	KI 2: Conclusion

Document for:	Discussion/Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes some principles for the conclusion of KI#2.

# 1.	Discussion and proposal

During the FS_eEDGE_5GC_Ph3 study item, some solutions proposed to leverage the NWDAF to obtain information about N6 delay and/or EAS load to enhance the EAS and local UPF (re)selection.

With respect to the EAS load, the NF load analytics defined in TS 23.288 are used to provide analytics about the 5G NFs, like AMF, SMF, UPF, but currently they are not used to provide information about the AS load. This is not the function of NWDAF and currently there is no NWDAF support for this type of determination. In addition, the benefits of introducing such feature in the NWDAF are questionable because the EAS load is assumed to change quite dynamically, while the NWDAF provides statistical values that are inherently more static.

Similarly, if the NWDAF was involved in providing analytics related to the N6 delay, it is unclear how beneficial this would be because the N6 delay typically varies quite dynamically.

Because of the reasons above it is proposed to exclude the NWDAF from the enhancement for EAS and local UPF (re)selection.

It is proposed to agree the following changes vs. TS 23.700-49:

# >>>>BEGINNING OF CHANGES<<<<

## 8.2	Conclusion for KI#2: Enhancement of EAS and local UPF (re)selection

The following principles are recommended in normative work for KI #2: Enhancement of EAS and local UPF (re)selection

-	SMF selects local PSA UPF considering N6 delay, when available.

Editor's note:	Whether SMF collects N6 delay measurements from the L-PSA UPF or from the AF is FFS

Editor's note:	Whether EAS load can also be used by the SMF/EASDF is FFS.

-	N6 delay between L-PSA UPF and EAS is measured by leveraging existing mechanisms (e.g., defined by IETF, PING, TWAMP, OWAMP, etc.)

-	Interaction between AF and 5GC may be needed to enable the measurement

Editor's note:	Details of such interaction are FFS.

-	The NWDAF shall not be involved in enhancements for EAS and local UPF (re)selection.

# >>>>END OF CHANGES<<<<