3GPP TSG-WG SA2 Meeting #163	S6-2406090

Jeju Island, South Korea, 27th – 31st May 2024	(revision of S2-240xxxx)

Source:	Samsung

Title:	Conclusion for KI#1

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: This pCR proposes principles for conclusion of KI#1.

1. Introduction/Discussion

This pCR propose principles to be included in the conclusion for KI#1. The proposed principles are based on the solution #1, #5, and #26.

2. Text Proposal

It is proposed to introduce the following changes in TR 23.700-49 v0.3.0.

# * * * * First change * * * *

8.1	Conclusion for KI#1

The following principles are concluded for the normative work.

-	The selection of I-SMF for local edge offloading control is performed by AMF based on the subscription data and UE location.

NOTE: 	I-SMF for local edge offloading control can be selected and inserted even if the UE is inside the service area of the SMF.

-	I-SMF for local edge offloading control performs EASDF configuration.

-	I-SMF for local edge offloading control retrieves EDI directly from NEF.

-	I-SMF for local edge offloading control obtains from PCF via SMF the local offloading management policy, which contains IP range(s) and/or FQDN(s) allowed to be offloaded to the local part of DN.

-	I-SMF for local edge offloading control selects local UPF(s).

-	I-SMF for local edge offloading control obtains SM policy related to the AF traffic influence request related to local offloading traffic under the control of the I-SMF.

-	I-SMF for local edge offloading control sends the notification for the UP path management event. It may require interaction among SMF, I-SMF and AF, if the runtime coordination between AF and 5GC is set up with respect to the AF acknowledgement to be expected.

# * * * * End of changes * * * *