SA WG2 Meeting #163	S2-2406105

Jeju, South Korea , May 27-31, 2024	(was S2-240XXXX)

Source:	Ericsson, Nokia, AT&T

Title:	KI#1: Conclusion on solution principles

Document for:	Discussion/Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes an interim conclusion for KI1.

# Proposal

It is proposed to accept the following changes to TR 23.700-77.

FIRST CHANGE

## 8.2	Principles for normative work of KI#1

For KI#1 on "Extensible IMS mechanism supporting IMS events in the context of DC communication" the following solution principles are proposed:

- The event SUBSCRIBE/NOTIFY framework must be extensible and shall support events that are IMS DC related and non-IMS DC related.

- The event SUBSCRIBE/NOTIFY framework must support events that are subscriber specific and non-subscriber specific. A subscriber specific event is e.g. an event like “user A has registered to the network or has established a call”, a non-subscriber event is e.g. an event like “a user has called number 800-123456” where 800-123456 is a service number.

- Subscription requests that are subscriber specific are sent to HSS which handles registered and unregistered UEs and ensures that a subscription is persistent when UE registers, deregisters and re-registers in IMS. That is, the HSS stores the subscription request and sends the subscription request whenever an IMS AS instance supporting the requested event is assigned to the UE. The information about the subscription events stored in HSS is only relevant and consumed by HSS.

- The IMS AS instance assigned to a UE during IMS registration shall be registered in HSS.

- Subscription requests that are non-subscriber specific are sent directly to all IMS AS instances.

- For purposes of optimization and following 5GC exposure principles, any Notification related to a subscription is sent directly to the NF (e.g. NEF or AF) that triggered the subscription request using a new Nimsas_ImsEE_Notify service operation.

- NEF and AF use a new Nhss_ImsEE_xxx SBI service to manage subscription requests (e.g. subscribe/unsubscribe) via HSS for subscriber specific IMS DC and non-IMS DC related events. NEF and AF may use a new Nimsas_ImsEE_xxx SBI service to manage bulk subscription requests with IMS AS(s).

NOTE 1: 	The need to manage bulk subscription requests with IMS AS(s) is determined during the normative phase.

- If there is more than one HSS deployed in the PLMN, NEF and AF discover the appropriate HSS instance(s) for a given user via NRF.

- Existing SBI Nhss_ImsUECM service used for S-CSCF registration over N70 is enhanced to allow the registration of the IMS AS instance in HSS over N71. A new Nimsas_ImsEE_xxx SBI service is defined to manage subscription requests with the IMS AS for subscriber specific IMS DC and non-IMS DC related events.

NOTE 2: 	The need to specify an Sh Diameter version of these procedures is determined during the normative phase.

- Events supported by an event producer are discoverable via NRF.

NOTE 3: 	Potential events to be exposed are: user registration/deregistration, call establishment/updating/answering/termination by a user, call established/updating/termination to a certain number (served by PLMN or not), BDC or ADC (with certain app ID) establishment/termination.

NOTE 4:	Events like application download is not supported in Release 19 but the event “ADC establishment for a specific application” is supported.

- Subscription requests to IMS events shall be optimized; event categories shall be used to group a collection of individual events in an event category for example “Telephony” or “DC” event category (e.g. DC events as defined in Table AA.2.4.2.2-1 of TS 23.228 [5]).

END OF CHANGES