3GPP TSG-WG SA2 Meeting #163	S2-2406176

# Jeju, Korea, May 27 – 31, 2024					                                     (revision of S2-2405671)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5356 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slicing for Indirect Network Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_NetShare |  |  |  |  |  | Date: |  |  | 2024-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case of Indirect Network Sharing, how to handle Network Slicing should be considered. Compared to roaming case as defined in the clause 5.15.6, for Indirect Network Sharing, the broadcasted PLMN IDs include the PLMN IDs representing participating operators and a UE from participating operator selects a PLMN ID representing the participating operator. Therefore, If the broadcasting PLMN ID which represents the participating operator is the one of the SUPI, the UE constructs Requested NSSAI based on the descriptions of non-roaming case. And the S-NSSAI from the Serving PLMN (i.e. hosting operator) sent to the UE should only include S-NSSAI of participating operator’s value (i.e. HPLMN S-NSSAI). After receiving the S-NSSAI of participating operator’s value from UE, the serving AMF determines the S-NSSAI of hosting operator’s value (i.e. VPLMN S-NSSAI) based on the S-NSSAI mapping agreement between hosting operator and participating operator and proceeds the relevant procedures involving network slicing based on the roaming case as defined in the clause 5.15.6. If the broadcasting PLMN ID which represents the participating operator is not the one in UE’s SUPI, the network slicing handling including UE and network shall comply with the roaming case. It is proposed to clarify the differences of Network Slice handling compared to the roaming case for Indirect Network Sharing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description on Network Slicing for Indirect Network Sharing to clarify the network will treat slicing as roaming case while the UE shall treat it as non-roaming in Indirect Network Sharing If the broadcasting PLMN ID which represents the participating operator is the one of UE’s SUPI. Otherwise, it is the same as HR roaming. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Network Slicing for Indirect Network Sharing is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | Revision 3: In case of Indirect Network Sharing, the differences of Network Slicing handling depend on the broadcasting PLMN which represents the participating operator: - If the selected PLMN ID is that of the UE’s SUPI(i.e. when the broadcast PLMN ID which represents the participating operator is the PLMN ID which is derived from UE’s SUPI), the network slicing handling for UE behaviour follows the non-roaming case and the network slicing handling for network behaviour follows the roaming case. - For other cases (e.g. the broadcast PLMN ID which represents the participating operator is included in the list of equivalent PLMNs), the network slicing handling including UE and network shall comply with the roaming case. The revision 3 proposes to clarify the Network Slicing handling. The descriptions endorsed in the S2-2405671 are highlighted in yellow. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.18.5	Network Sharing and Network Slicing

As defined in clause 5.15.1, a Network Slice is defined within a PLMN or SNPN. Network sharing is performed among different PLMNs and/or SNPNs.

In the case of5G MOCN, each PLMN or SNPN sharing the NG-RAN defines and supports its PLMN- or SNPN- specific set of slices that are supported by the common NG-RAN.

In the case of Indirect Network Sharing, the differences of Network Slicing handling is perfomed as the follows:

-	If, in an Indirect Network Sharing area, the selected PLMN ID is that of the UE’s SUPI (i.e. when the broadcast PLMN ID which represents the participating operator is the PLMN ID which is derived from UE’s SUPI), the network slicing handling shall be performed by the UE based on the descriptions of non-roaming case as defined in clause 5.15. From the network side, the network slicing handling shall be performed as defined in clause 5.15.6. The changes compared to baseline operations are as follows:

-	During Registration procedure, the Requested NSSAI from the UE only contains HPLMN S-NSSAIs. After receiving the Requested NSSAI of participating operator (i.e. HPLMN S-NSSAIs) from UE, the serving AMF determines the corresponding S-NSSAIs of hosting operator (i.e. VPLMN S-NSSAIs) based on the S-NSSAI mapping agreement between hosting operator and participating operator. The serving AMF further checks whether the corresponding S-NSSAIs of hosting operator are supported by the shared RAN. If not the serving AMF rejects the Registration Ruquest. Otherwise the serving AMF continues the Registration procedure and sends to the UE Allowed NSSAI and Configured NSSAI only containing HPLMN S-NSSAI(s) (i.e. without providing slice mapping information) and sends to the shared RAN Allowed NSSAI containing the S-NSSAIs of hosting operator (i.e. VPLMN S-NSSAIs).

-    During PDU Session Establishment procedure, the UE only includes the HPLMN S-NSSAI in the PDU Session Establishment Request. After receiving the S-NSSAI of participating operator from UE (i.e. HPLMN S-NSSAI), the serving AMF uses the S-NSSAI of hosting operator (i.e. VPLMN S-NSSAI) as "the S-NSSAI belonging to the Allowed NSSAI that maps to (a) using the mapping of the Allowed NSSAI to HPLMN S-NSSAIs" as defined in clause 5.15.6.

NOTE 1:  In the case of Indirect Network Sharing, the shared RAN only needs to support the S-NSSAI of Hosting operator and select AMF as described in clause 5.15.5.2.1. This does not mandate the shared RAN to be provisioned with the mapping from participating operator’s network NSSAI values to hosting operator’s network NSSAI values.

-    For other cases, i.e. the selected PLMN ID is not that of the UE’s SUPI (e.g. the broadcast PLMN ID which represents the participating operator is included in the list of equivalent PLMNs), the network slicing handling including UE and network shall comply with the roaming case as defined in the clause 5.15.6.

NOTE 2:  Which broadcast option to be used and whether to broadcast one or more PLMN ID(s) for each participating operator is determined based on the agreement between the hosting operator and participating operator.

# * * *  End of changes * * * *