3GPP SA WG2 Meeting #163	S2-2406186

May 27 – 31, 2024, Jeju, South Korea

Source:	Ericsson

Title:	KI#2: Conclusions

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: The contribution proposes a discussion of the solution alternatives for KI#2 and conclusions for the normative work.

# Discussion

This contribution provides conclusion to KI#2 from TR 23.700-49, clause 5.2, “Enhancement of EAS and local UPF (re)selection”.

There are 9 solutions for the KI#2 (solutions #8-#16)). Out of them, 7 solutions (solution #8, #9, #10, #11, #12, #15, #16) consider N6 latency, and 5 solutions (solution #9, #12, #13, #14, #15) consider EAS load.

Regarding the N6 latency consideration in the EAS discovery and (re-)selection, the main issue is whether the SMF should receive the N6 delay information from the AF or the L-PSA (UPF). The former alternative requires an additional complexity because the AF should continuously deliver the QoS monitoring requirement for those EAS IP addresses that are active at a given time. It is thus preferred that the AF provides the delay information rather than the L-UPF (PSA).

A complexity with performing the N6 latency measurements is that the other party (i.e., the one not performing the N6 measurements) should provide additional assistance information (e.g., measurement endpoint information) for N6 delay measurements. In the case when the AF provides the delay information to the SMF this can be avoided in the following way: instead of the N6 latency, the AF provides the end-to-end latency to the SMF and the SMF then subtracts the 5GC latency between the UE and L-UPF to infer the N6 delay. Such a solution for measuring and providing end-to-end latency has been proposed in SA6 SEAL, see TS 23.433, clauses 4.3 and 9.7. The SMF could use the data transmission query from 9.7.2.2 TS 23.433 to fetch the information, or SA2 could define some new means for the AF to provide the e2e delay to SMF (e.g., via NEF).

Observation 1: Regarding the N6 latency consideration in the EAS discovery and (re-)selection, the solution with smallest impact is when the AF provides the end-to-end delay to SMF.

Regarding the EAS load, the main issue is how the EAS load should be considered in the EAS (re-)discovery. A few solutions propose that the EAS load information is conveyed by the AF to the NEF and then to the SMF.

- Cloud-internal load optimization mechanisms (load balancing, auto-scaling) are already designed to avoid server overload.

- For the remaining cases of EAS overload protection, there is a simple and efficient service side centralized overload management already in place. The method is to manage the records in the DNS server to return the DNS response to the UE a list of EASes that are not overloaded. This method may also be used for load balancing among the EASes. We do not see how a distributed overload management by the different SMFs accessing the same EAS could have a superior performance:

  - Centralized vs distributed overload management: there may be different SMFs accessing the same EAS IP, and each of them should receive the required monitoring information and implements an overload protection mechanism.

  - Reporting of EAS load by the AF via NEF to the SMF has an inherent latency, thus smaller load thresholds would have to be set even for the real-time measurements to avoid potential overload, which results in lower overall EAS utilizations then when the overload protection were done at the service side.

  - It is unclear how the two mechanisms would interfere if both were in place. The DNS-based method may also be used for load balancing among the different EASes. If the SMF manipulated the list of EASes that were put in a specific order by the service DNS, this would result in a different EAS usage pattern that the service network expects. However, the SMF does not have EAS capacity information to take over the load balancing task.

Observation 2: Efficient server-side EAS overload protection mechanisms are already in place. None of the solutions for EAS discovery considering EAS load describe the benefits vs. and interworking with the current solutions.

# Proposal

It is proposed to add these changes to TS.  23.700-49

***************** 1st change *****************

## 8.2	Conclusion for KI#2

The following principles are recommended in normative work for KI #2: Enhancement of EAS and local UPF (re)selection

-	SMF selects local PSA UPF considering N6 delay, when available.

-	SMF collects delay measurement information from the AF. For the normative phase it is proposed to leverage on the SA6 enabler functions (e.g., clauses 4.3 and 9.7 of TS 23.433 [x]) as far as possible with minimal changes, to provide the lowest impact on the current architecture.

-

-	The SMF does not consider EAS load information in the EAS and Local UPF (re)selection. It is assumed that EAS overload protection is achieved at the service side.

-	Solutions assuming NWDAF assistance should be pursued only if there is no impact on the current architecture and analytics.

***************** 2nd change *****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS)".

[5]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[6]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[7]	IETF RFC 792: "Internet Control Message Protocol".

[8]	IETF RFC 4443: "Internet Control Message Protocol (ICMPv6) for the Internet Protocol Version 6 (IPv6) Specification".

[9]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[10]	IETF RFC 4656: "A One-way Active Measurement Protocol (OWAMP)".

[11]	IETF RFC 5357: "A Two-Way Active Measurement Protocol (TWAMP)".

[12]	IETF RFC 8762: "Simple Two-Way Active Measurement Protocol".

[13]	IETF RFC 9484: "Proxying IP in HTTP".

[x]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

***************** End changes *****************