SA WG2 Meeting #S2-163	S2-2406218

27 - 31 May, 2024, Jeju, South Korea	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0137 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification on the service exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the LS (S2-2405850, SP-240497) from SA plenary and attached CRs, the service exposure to client UE via 5GC (including both U-plane and C-plane) is removed. In the clause 5.6.1, some descriptions need to be modified. “The Ranging/SL Positioning service request includes the identifiers of the two or more UEs. When the service exposure is via 5GC, …” In the existing spec, the Ranging/SL Positioning service request is sent by UE. The AF send the LCS service request (see 23.273 clause 6.20.3 step 1). Therefore, it proposes to either remove the sentence “When the service exposure is via 5GC,”, or clarify “exposure is to AF”. And “exposure through 5GC U-plane” needs to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the service exposure to client UE via 5GC description according to LS (S2-2405850). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not align with SA LS and approved CRs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.6	Ranging/SL Positioning service exposure

### 5.6.1	General

Ranging/SL Positioning service can be exposed to the authorized SL Positioning Client UE, 5GC NF or AF to obtain the relative position or distance/direction result between two or more UEs capable of Ranging/SL Positioning.

Ranging/SL Positioning service also can be used by the authorized SL Positioning Client UE, 5GC NF, AF or the LCS client to obtain the absolute position of Target UE if the 5GC NF, AF or the LCS client determines that Ranging/SL Positioning is applicable.

The Ranging/SL Positioning service request includes the identifiers of the two or more UEs. When the service exposure is via 5GC to AF, the GMLC will translate the target UE's identifiers into SUPI. When the service exposure is via PC5, the identifiers can be Ranging/SL Positioning application specific Application Layer IDs.

The request may include the required QoS, the required location results (e.g. absolute locations, relative locations or distances and/or directions related to the UEs as defined in clause 4.4).

The service request may further include periodic or trigger event parameters, e.g. the time interval between successive location reports and the total number of reports, the trigger events, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates shall be included in event reports, and whether only one location report is required or more than one. Trigger events can be;

-	the relative position/distance between at least one pair of UEs of the indicated UEs is less than the threshold;

-	the relative position/distance between at least one pair of UEs of the indicated UEs exceeds the threshold;

-	the relative positions/distances between all UEs are less than the threshold; or

-	the relative positions/distances between all UEs exceed the threshold.

END OF CHANGES