SA WG2 Meeting #162	S2-2406237

May 27 – 31, 2024, Jeju, Korea

Source:	Ericsson

Title:	KI#1.x: Updates to Solution #1.1

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes updates to Solution #1.1.

# Discussion

This solution update incorporates the following changes to the Solution#1.1:

- 	The first alternative with No AMF impact is removed.

- 	New parameter is added to SM subscription data to allow DS per DNN/S-NSSAI

- 	New UE policy, DS policy, is introduced for identifying the SUPIs which are linked for DS purposes.

- 	AMF includes the second SUPI in the PDU Session Create Context request to SMF to allow SMF to find the PDU Session of the linked SUPI

- 	EPS interworking section is left FFS since it is no longer aligned with the updated solution.

# Proposal

It is proposed to update TR 23.700-54 as follows:

**** First Change ****

## 6.1	Solutions for DualSteer

### 6.1.1	Solution #1.1: Supporting dual-steer connection re-using certain ATSSS functionality

#### 6.1.1.1	Overall Description

##### 6.1.1.1.1	Introduction

The proposed solution is based on the following principles:

-	The impact to AMF is kept minimal. This enables the DS functionality when the VPLMN makes minimal enhancements to the AMF.

-	Either of two SUPIs can be used by the UE to first register or request the PDU session establishment.

-	The solution considerers the DS device to act as either one or two UEs towards the network but does not discuss the UE architecture. The requirements on the DS device are listed in the impact section.

-	The traffic switching and steering is based on using a common SMF and UPF. Traffic splitting is not supported.

-	UPF treats the DS PDU session in a similar manner as an MA PDU session. However, establishment and management of the DS PDU Session is different than that of the MA PDU session.

NOTE:	How the UE decides to register to a second 3GPP access network and how the UE selects this 3GPP access network is assumed to be addressed by other solutions.

##### 6.1.1.1.2	Architectures

This solution is based on providing a common anchor (UPF) and SMF inside the 5GC for the DS traffic from both 3GPP access networks. Non-roaming and roaming architectures are shown below.

Figure 6.1.1.1.2-1: Non-roaming dual steering architecture

Figure 6.1.1.1.2-2: Dual steering architecture with roaming in one access

Figure 6.1.1.1.2-3: Dual steering architecture with roaming in both accesses with common VPLMN

Figure 6.1.1.1.2-4: Dual steering architecture with roaming in both accesses with two different VPLMNs

##### 6.1.1.1.3	Registration

Alt 1: Void.

Limited AMF impact: The AMF may indicate (visited) network's support for DS PDU session in the initial Registration Accept message to UE.

##### 6.1.1.1.4	Policy

URSP Rules: To support establishment of DS PDU session, the URSP rules are extended. Specifically, the Access Type preference in Route Selection Descriptor is extended to include a Dual Steering indication.

DS Policies:

This UE policy contains DS SUPIs. The DS SUPIs is used by the DS device to identify the two USIMs that can be used for DS.

Table 6.1.1.1.4-1: DS SUPIs

| Information Name | Description |
| --- | --- |
| DS SUPIs | A two-tuple containing two DS SUPIs |

Editor's note: Whether additional UE policies related to RAT selection for a selected PLMN are needed is FFS and depends on SA1 outcome.

PCC rules: Considering that data splitting is not supported, the allowed steering modes are Active-Standby and Smallest Delay (only if simultaneous transmission is supported). For the Active-Standby mode, the Access type and connected PLMNs can be important. Therefore, the MA PDU session Control part of the PCC rules is enhanced as follows:

-	A new part, DS control, is added to the PCC rules with the following information.

-	An Access descriptor which includes a 4-tuple (Access1-type, Access1-PLMN-ID, Access2-type, Access2-PLMN-ID), identifying Access1 and Access2. The access type may be one of the following: T-NR, GEO, LEO, LTE, non-3GPP, other.

-	The fields charging/monitoring key  include charging/monitoring key for Access2.

Table 6.1.1.1.4-3: Enhancement on the PCC rule for Dual Steering

| Information Name | Description |
| --- | --- |
| DS Control | This part defines information supporting control of Dual Steering |
| Application descriptors | Identifies the application traffic (NOTE 1) |
| Access descriptor | Identifies the access type and PLMN via a 4-tuple as follows: (Access1-type, Access1-PLMN-ID, Access2-type, Access2-PLMN-ID) (NOTE 2) |
| Steering Functionality | Indicates the applicable traffic steering functionality: Values "MPTCP functionality", "ATSSS-LL functionality", "MPQUIC functionality" |
| Steering Mode | “Shortest delay” (only if simultaneous transmission is supported) or “Active-Standby” (NOTE 3) |
| Transport Mode | (NOTE 1) |
| Charging key forAccess2 | (NOTE 1) |
| Monitoring key for Access 2 |  |
| NOTE 1: Similar to how it is defined for MA PDU session in clause 6.3.1 of TS 23.503 [5]. NOTE 2: The access type can be one of {T-NR, GEO, LEO, LTE, non-3GPP, other}. NOTE 3: As an example, the Active Standby mode is described as “Active: Access1, Standby: Access2”, where Access1 and Access2 can be identified by the Access Descriptor. |  |

The PCF should subscribe to changes of RAT, Access Type, and PLMN to SMF for both SUPI1 and SUPI2 and it may update the PCC rules based on the notifications.

Editor's note:	Further description on how to handle a single SM policy association related to two SUPIs, is FFS.

DS Rules: To enforce DS policies in the UL direction, the DS rules are defined to provide policies for DualSteer traffic steering and switching and enhanced with a new Access Descriptor parameter (compared to the ATSSS rules). This is the same 4-tuple as described in Table 6.1.1.1.4-3 and guides the DS device to identify which one of the accesses is Access1 and which one is Access2.

Table 6.1.1.1.4-4: DS rules structure

| Information name | Description |
| --- | --- |
| Rule identifier | NOTE 1 |
| Rule Precedence | NOTE 1 |
| Traffic Descriptor | NOTE 1 |
| Application descriptors | NOTE 1 |
| IP descriptors | NOTE 1 |
| Non-IP descriptors | NOTE 1 |
| Access descriptor | NOTE 2 |
| Access Selection Descriptor | NOTE 1 |
| Steering Mode | NOTE 2 |
| Steering Functionality | NOTE 1 |
| Transport Mode | NOTE 1 |
| NOTE 1: The same clause 5.32.8 of TS 23.501 [3]. NOTE 2: The same as in Table 6.1.1.1.4-3 |  |

N4 Rules: The same mechanism for MA PDU session can be implemented for DS PDU session, i.e., the SMF generates two FARs that direct the traffic to the two accesses and one MAR that points to one of the FARs depending on whether the Active path is chosen for downlink traffic or the standby one (in the Active-Standby mode).

If the device does not support simultaneous transmission, it should provide the Access Availability/Unavailability reports to the PMF and both UE and UPF change the access after transmitting/receiving the acknowledgement from UPF. For example, if the report indicates “The active access is Unavailable”, both DS device and the UPF change to the standby access.

##### 6.1.1.1.5	Subscription and UE context data in UDM/UDR

The connection between the two SUPIs is maintained in UDM. For this purpose, a new field, DS SUPI, is added in the Access and Mobility Subscription Data for both SUPI1 (containing SUPI2) and SUPI2 (containing SUPI1).

A new parameter, DS Correlation ID, which is used to connect two PDU session requests is added to SMF registration information and UE context in SMF data in UDM/UDR.

NOTE: 	If the DS device can provide the same PDU Session IDs for both PDU Sessions, with the PDU Session ID being unique for both SUPI1 and SUPI2, a new DS Correlation ID should not be needed.

To indicate if the PDU session with Dual steering is allowed for certain DNN/S-SNSSAI, the Session Management Subscription data is enhanced as follows.

Table 6.1.1.1.4-5: Enhancement to the Session management subscription data in UDM

| Session Management Subscription data (data needed for PDU | For each DNN in S-NSSAI level subscription data: |  |
| --- | --- | --- |
| Session Establishment) | DS information | Indicates whether DS PDU session establishment is allowed. |

How these new parameters are used is described on the procedures below.

##### 6.1.1.1.6	Session management

This solution is based on creating a DS PDU session that is anchored in a common UPF and managed by a common SMF. To achieve this:

-	Two separate PDU session Establishment requests are sent by the UE (one for each SUPI).

-	In the PDU session Establishment requests, UEs provides a new parameter (DS Correlation ID) to enable identifying which two PDU session requests correspond to the same DS PDU session.

-	Both AMFs select the same (H-)SMF.

Selection of common SMF is done via DS Correlation ID matching by AMF.

#### 6.1.1.2	Procedures

##### 6.1.1.2.1	Session management

Note: 	In this clause we consider two separate UDMs to manage the subscription data of SUPI1 and SUPI2, however, since both UDMs belong to the HPLMN, it can be assumed that the subscription data of both SUPIs are managed in a single UDM.

Figure 6.1.1.2.1-1: Dual steering PDU Session Establishment

1.	 DS device registers to 5GC through AMF1 using SUPI1 and receives DS URSP rules including the DS policies with DS SUPIs, which provide information of which SUPIs (SUPI1 and SUPI2) which can be used in a DS session.

2.	DS device sends PDU Session Establishment request with Request Type initial request, using SUPI1 and includes in the MM part of the NAS message and the N1 SM container a new parameter, DS Correlation ID. The DS Correlation ID is generated by the DS device and should uniquely identify the DS PDU Session across both UEs.

3.	The AMF performs SMF discovery and selection via NRF and selects a SMF that supports the DS functionality.

4.	AMF sends a Nsmf_PDUSession_CreateSMContext request to the SMF and adds SUPI2, which is included in the DS SUPI field in the Access and Mobility Subscription data.

5	SMF retrieves the SM subscription data from UDM. DS information for SUPI1 indicates DS PDU session establishment is allowed.

6.	SMF establishes a SM Policy Association and, in addition to SUPI1, SMF includes the DS SUPI (SUPI2) in the request. The PCF provides the PCC rules for DS PDU session, including DS control information.

7.	The SMF establishes an N4 session with the UPF. SMF generates and sends to UPF N4 rules similarly as for a MA PDU Session.

8.	The SMF Registers the PDU Session with UDM. In addition, it includes the DS Correlation ID as new parameter in the registration request.

9.	If the DS device is not already registered with SUPI2 through a second access, it registers in this step and receives DS URSP rules.

Editor's note:	How step 9 is triggered is assumed to be addressed by other solutions.

10. This step is the same as step 2 with the request type set to Existing PDU session. . The DualSteer device sends the same DS Correlation ID and the same DNN/ S-NSSAI as in step 2.

11. AMF2 uses the DS SUPI field in the Access and Mobility Subscription data of SUPI2 to discover SUPI1 and gets from UDM the UE context in SMF data for SUPI1 and selects SMF based on DS Correlation ID.

12. Similar to step 4 but for SUPI2, providing in this case SUPI1 in the DS SUPI field from the Access and Mobility Subscription data.

13.	Similar to step 5 but for SUPI2.

14.	SMF triggers SM policy association update request to update the policy information for the PDU Session and includes DS SUPI (SUPI1). PCF may modify the PCC rules based on the access type and PLMN of the SUPI2.

15.	SMF modifies the N4 rules so that an additional N3/N9 tunnel is established between the second access and the UPF.

16.	The same as steps 11-21 of figure 4.3.2.2.1-1 of TS 23.502 [4], with the difference that in step 16c the SMF includes DS Correlation ID in the registration of the PDU session as in step 8.

Editor's note:	Description of additional procedures is FFS.

Editor's note: Interworking with EPS is FFS

#### 6.1.1.3	Impacts on services, entities and interfaces

DS device: Registering with two separate SUPIs; supporting steering and switching across two PDU sessions associated with two SUPIs; providing a unique DS correlation ID in the N1 SM container in the PDU Session Establishment requests for both SUPIs. Handling enhanced URSP and DS rules.

AMF: Selecting SMF with DS capability, Selecting SMF based on DS Correlation ID. Providing indication of support for DS PDU session in initial Registration Accept message. Including DS SUPI in PDU Session Create Context request to SMF.

N4 enhancements: Support for providing two SUPIs to UPF.

UPF: Existing support for MA PDU Sessions is re-used (e.g. the use of MAR pointing to two FARs). N4 enhancements as described above.

SMF: Creating N4 rules and DS rules for DS PDU session, including N4 enhancements as described above.

UDM/UDR: Adding a new parameter, DS SUPI, to UE Access and Mobility subscription data. Adding a new parameter to PDU Session context, DS Correlation ID. Adding DS Information parameter to SM subscription data.

PCF: Handle a SM policy association related to two SUPIs. Creating PCC rules with DualSteer information.

NRF: Adding a new parameter for SMF discovery; Capability of the SMF to support DualSteering.

**** End of Changes ****