3GPP SA WG2 Meeting #163	S2-2406267

Jeju, South Korea, 2024-05-27 – 2024-05-31

Source:	Ericsson

Title:	Conclusion of KI#4

Document for:	Approval

Agenda Item:	#19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: . Proposing conclusions for KI#4

# 1	Discussion

The following table summarizes different aspects of solutions for KI#4 and provides an evaluation of them mostly based on their impact.

Table: Summery/Evaluation of Solutions for KI#4

| Solution number | Sol #30 | Sol #31 | Sol #32 | Sol #33 | Sol #34 |
| --- | --- | --- | --- | --- | --- |
| Storing device QoS requirements | Inside AF server | In UDR via AF/NEF | In UDR | NA | In UDR via AF/NEF |
| Provisioning QoS requirement to PCF | AF/NEF pushes the QoS requirements to PCF | Querying UDR via device identifier | Querying UDR via device identifier | NA | Querying UDR via device identifier |
| Provisioning the device traffic identifier to PCF | AF/NEF pushes the traffic identifier to PCF | Via NAS singling in PDU session modification request | Via NAS singling in PDU session modification request | NA | Via NAS |
| Limiting the number of active devices | Yes, Configuration in AF | Not specified | Yes, via Providing N3DBU IDs during registration | Yes, AMF counts and checks the number of devices | Not specified |
| Sharing PDU session between devices | Yes | Yes | Yes | NA | Yes |
| Supporting MAC randomization | Yes | Yes, via IRM | Not specified |  | Not specified |
| URSP rules | Re-using PIN-ID+ URSP provisionen via AF | Re-using Connectivity Group ID | New TD, N3DBU ID + URSP provisionen via AF | NA | No URSP rules. |
| AMF/VPLMN impact | No impact | minimal, selecting SMF with the right capability | Yes, providing N3DBU IDs in registration. Selecting SMF with right capability | Yes | minimal, PCF selection |
| Other 5GC impacts | NEF, PCF | NEF, UDR, SMF, PCF | SMF, UDM, UDR, PCF, NEF | UDM | AMF, SMF, PCF, UDM,UDR |
| Other impacts | AF server to receive and store the device information. UE/5G-RG supporting a client program. | AF needs to provide device information to UDR, UE/5G-RG needs to handle NAS modifications | UE/5G-RG to handle NAS signaling for registration | UE/5G-RG | AF, UE/5GRG |
| Evaluations | low impact on 5GC and no impact on AMF/VPLMN. Requiring a client program in UE/5GRG and AF server | Minimal impact on AMF/VPLMN Impacting NAS signaling and relatively high 5GC impact/ requiring AF server | High 5GC impact. Impacting AMF/VPLMN and NAS signaling. Limiting the number of devices with differentiated services. | High impacts on AMF/VPLN to limit the number of connected devices | no impact on AMF/VPLMN if homogeneous implementation of SMF/PCF is considered. Impacting NAS and 5GC |

Among the solutions, Solutions 32 and 33 require considerable AMF/VPLMN impact for ensuring that the number of devices behind UE 5G-RG is below a maximum number, which is not desirable. It is important that AMF impact is kept low to ensure that the feature can be used in roaming scenarios without significant impact on VPLMN.

Solutions #31 and #34 are close to ach other and have lower impact on 5GC. Compared to Solution #30 they have the following differences:

- The data is stored in UDR with help of AF instead of storing data in the AF server.

- The request for differentiated service comes from NAS signaling instead of AF.

Since both Solutions #31 and #34 require an AF to store the QoS parameters into UDR and possibly interact with the user, additional impact in the 5GC and NAS signaling is not justified. In addition, Solution #30, as opposed to Solutions #32 and #34 supports limiting the number of devices with differentiated services.

Therefore, should the study be concluded at this stage, it is proposed to use the principles of Solution #30.

# 2.	Proposal

For KI4, there are a number of solutions that require updates and a number of solutions that are not added to TR 23.700-32. Therefore, it is proposed to postpone the conclusion. Nonetheless, if the study should conclude at this stage we propose the following changes.

***************** Start changes (All new) *****************

## 7.X	Evaluation of solutions for KI#4:

Table 7.X.1: Summery/Evaluation of Solutions for KI#4

| Solution number | Sol #30 | Sol #31 | Sol #32 | Sol #33 | Sol #34 |
| --- | --- | --- | --- | --- | --- |
| Storing device QoS requirements | Inside AF server | In UDR via AF/NEF | In UDR | NA | In UDR via AF/NEF |
| Provisioning QoS requirement to PCF | AF/NEF pushes the QoS requirements to PCF | Querying UDR via device identifier | Querying UDR via device identifier | NA | Querying UDR via device identifier |
| Provisioning the device traffic identifier to PCF | AF/NEF pushes the traffic identifier to PCF | Via NAS singling in PDU session modification request | Via NAS singling in PDU session modification request | NA | Via NAS |
| Limiting the number of active devices | Yes, Configuration in AF | Not specified | Yes, via Providing N3DBU IDs during registration | Yes, AMF counts and checks the number of devices | Not specified |
| Sharing PDU session between devices | Yes | Yes | Yes | NA | Yes |
| Supporting MAC randomization | Yes | Yes, via IRM | Not specified |  | Not specified |
| URSP rules | Re-using PIN-ID+ URSP provisionen via AF | Re-using Connectivity Group ID | New TD, N3DBU ID + URSP provisionen via AF | NA | No URSP rules. |
| AMF/VPLMN impact | No impact | minimal, selecting SMF with the right capability | Yes, providing N3DBU IDs in registration. Selecting SMF with right capability | Yes | minimal, PCF selection |
| Other 5GC impacts | NEF, PCF | NEF, UDR, SMF, PCF | SMF, UDM, UDR, PCF, NEF | UDM | AMF, SMF, PCF, UDM,UDR |
| Other impacts | AF server to receive and store the device information. UE/5G-RG supporting a client program. | AF needs to provide device information to UDR, UE/5G-RG needs to handle NAS modifications | UE/5G-RG to handle NAS signaling for registration | UE/5G-RG | AF, UE/5GRG |
| Evaluations | low impact on 5GC and no impact on AMF/VPLMN. Requiring a client program in UE/5GRG and AF server | Minimal impact on AMF/VPLMN Impacting NAS signaling and relatively high 5GC impact/ requiring AF server | High 5GC impact. Impacting AMF/VPLMN and NAS signaling. Limiting the number of devices with differentiated services. | High impacts on AMF/VPLN to limit the number of connected devices | no impact on AMF/VPLMN if homogeneous implementation of SMF/PCF is considered. Impacting NAS and 5GC |

***************** Next changes all new*****************

## 8.X	Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

To support differentiated services for devices behind UE/5G-RG, the following principles are proposed as conclusion:

-	The device information is stored in an AF.

-	The device information includes a mapping between device identifier and device traffic information that is used to identify the devices in the UP

-	The binding between non-3GPP devices and device identifier is performed locally at the UE/5G-RG, and how it is done is out of 3GPP scope.

-	A client program in UE/5G-RG updates the device information in the AF.

-	The client in the UE/5G-RG gets acceptance from the AF whether added device information was accepted, if it was not accepted then the UE/5G-RG provides default connectivity to the non-3GPP device.

-	The required QoS differentiation is provisioned via AF server by requesting an AF session with required QoS.

-	The device traffic information used to identify the devices in the UP can be port ranges or IPv6 address.

***************** End changes *****************