SA WG2 Meeting #163	S2-2406274

Jeju, Korea, May 27-31 2024

# Title:	KI#2: conclusions

# Source:	Nokia

# Document for:	Approval

Agenda Item:	FS_EnergySys_Ph2/19.4

Abstract:	this paper provides conclusions on KI#2

1. Introduction

Based on the outcomes of the Moderated email discussion we propose to provide an update to the conclusions for KI#2

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66.

# Proposed text

## 8.2	Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

For KI#2, the following enhancements on subscription and policy control are agreed as interim conclusions to support energy efficiency and energy saving as service criteria in the 5GS:

-	The following are supported:

-	Energy saving subscription information to allow the network to perform energy saving for the UE.

-	The PCF may take into account energy related information including subscription data from previous bullet when making policy decisions on the AM, SM and UE policies. Subscription information may include different levels of QoS (e.g  Session AMBR) per energy state in the NG-RAN, different URSPs per Preferred Energy settings at UE, RFSP.

- 		The PCF may Subscribe to analytics from NWDAF (see KI#3 for the supported analytics) or energy related spending limits with the CHF to enforce AM, SM policies. The PCF may also receive energy related information from SMF based e.g. on the information the SMF receives from NG-RAN/UPF (e.g. whether the UE is detected to be causing high energy consumption).

- 	The energy saving policies are supported in non-roaming case only in this release.

-	Subscribed network slices may be associated to Energy related system behaviour. A UE subscription e.g. may include only Energy saving slices or a mix of slices with different energy behaviours (i.e. network slices which are aimed at performance only and some aimed at delivering energy saving system behavior) or no energy saving network slices at all.

-

# End of proposed text