SA WG2 Meeting #163	S2-2406398

# 27 – 31 May 2024, Jeju, South Korea

Source: 	NTT DOCOMO

Title: 	KI#2 Conclusion

Document for: 	Approval

Agenda Item: 	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This contribution proposes the conclusion for KI#2.

1	Discussion

Based on the solutions documented in TR 23.700-45, this paper proposes a set of principles to be considered in the normative work for KI#2.

Those principles are derived based on the NWM discussion outcome, (See Rapporteur proposal: https://nwm-trial.etsi.org/#/documents/8866.)

2	Proposal

It is proposed to update TR 23.700-45 according to the following text.

********** First Change **********

# 8	Conclusions

## 8.1	Key Issue #2: Enabling provisioning of subscribers allowed to access CAG cell and managing access control by the CAG owner or an authorized administrator

It is agreed to consider the following principles for the normative work:

-	The NEF API, i.e. Parameter Provisioning specified in the clause 4.15.6 of TS 23.502 [3] is enhanced to support the 5G Femto CAG provisioning.

-	UE impacts shall be avoided.

-----------------------------------------------------

Alt#1:

-	The CAG-administrator provides CAG info via NEF in HPLMN of the target UE both for the roaming case and the non-roaming case.

-	AF redirection is not a part of the solution.

-	The procedure is to be written under an assumption that AF has found an appropriate NEF and is authorized to access the NEF.

-	NOTE needs to be added after the normative text to explain the limitation of the specification with regards to e.g., discovery of NEF and authorization at NEF.

-	The CAG-administrator-provided CAG info is stored in UDM/UDR in HPLMN of the target UE both for the roaming case and the non-roaming case.

-	UDM updates AMF for CAG info according to Rel-18 existing specification.

-----------------------------------------------------

Alt#2:

-	The CAG-administrator provides CAG info via NEF in PLMN that 5G Femto serves.

-	The CAG-administrator-provided CAG info is stored in an NF in PLMN that 5G Femto serves. For the non-roaming case, the NF is UDM/UDR. For the roaming case, the NF is V-UDR.

-	The details of procedures related to V-UDR are finalized in the normative work.

-	For the non-roaming case, UDM updates AMF for CAG info according to Rel-18 existing specification.

-----------------------------------------------------

-	AMF updates UE for CAG info according to Rel-18 existing specification.

********** End of Changes **********