3GPP TSG-SA WG2 Meeting #163	S2-2406526

Jeju, Korea, May 27 – May 31, 2024

Source: 	Thales(rapporteur)

Title: 	Introduction for conclusions on the architecture principles following the NWM discussions, for KI1

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes Introduction for conclusions on the architecture principles following the NWM discussions, for KI1.

# 1 Discussion

This paper summarizes architecture principles that can be deduced after the 2 rounds of NWM discussion (see https://nwm-trial.etsi.org/#/documents/8860).

Method used is to directly deduce conclusions from observations made, observations being made following choices of the majority (i.e.: the greatest number) of answers.

# 2 Proposal

It is proposed to agree below proposed changes to 23.700-29.

*** First Change (all new text) ***

# 8	Conclusions

## 8.X Introduction for conclusions on the architecture principles

This chapter summarizes architecture principles that can be deduced after the 2rounds of NWM discussion (see https://nwm-trial.etsi.org/#/documents/8860).

Method used is to directly deduce conclusions from observations made, observations being made following choices of the majority (i.e.: the greatest number) of answers.

### 8.X.y Introduction for conclusions on the architecture principles for KI#1

It is proposed to agree on the following principals:

For Cat1 solution type as described in §7 of TR23.700-29:

KI1_Cat1_P1: Solutions which do not propose an intermediate IWF or proxy that can hide the mobility of eNB/gNB that is on board the satellite, and therefore migration of moving eNB/gNB (disconnecting and reconnecting a moving eNB/gNB leaving an area controlled by the CN) shall be supported.

KI1_Cat1_P2: Support of a procedure to handle the N2 and S1 connections in the Core Network when the gNB/eNB leaves an area served by an AMF/MME (e.g. when setting over the horizon) is for RAN3 decision.

KI1_Cat1_P3: NB/eNB IP address change (if any) due to soft feeder link switch can be supported using existing procedures.

KI1_Cat1_P4: AMF/MME can treat the Mapped Cell IDs as per Rel-17.

For Cat2 solution type as described in §7:

KI1_Cat2_P1: Solutions which propose an intermediate IWF or proxy (called Intermediate GW for convenience) between the moving eNB/gNB and CN are to be considered as optional deployment option.

KI1_Cat2_P2: The intermediate GW shall play the role of “earth fixed”eNB/gNB towards the CN and the role of AMF/MME/UPF towards eNB/gNB.

KI1_Cat2_P3: The Intermediate GW shall use existing N2/N3/S1 interfaces to connect to CN.

KI1_Cat2_P4: The intermediate GW shall propagate existing R18 ULI information for NTN.

KI1_Cat2_P4: CN S1/N2 handover procedures must not be impacted.

KI1_Cat2_P5: If intermediate GW has no impact on specification, SA2 can document it in informative Annex in TS 23.501 [2] and TS 23.401 [5] during the normative phase of the work as a deployment option.

KI1_Cat2_P6: If intermediate GW is deployed as implementation choice it shall be transparent for CN side.

Other principals whatever the category:

KI1_P7: Scenario of hard feeder link switch can be addressed by existing procedures, data buffering in UPF can be discussed during normative phase.

KI1_P8: There is no need to introduce new RAT type for regenerative payload satellite access.

*** End Of Change ***