# SA WG2 Meeting #S2-163	S2-2406588

# 27 - 31 May, 2024, Jeju, South Korea

Title:	Reply LS to LS on GPSI and Application Layer ID Mapping

Response to:	Question on GPSI and Application Layer ID Mapping (S2-2405863/ C3-242616)

Release:	Release 18

Work Item:	Ranging_SL

Source:	SA2

To:	CT3

Cc:	CT4

Contact Person:	Yaxin Wang

wangyaxin1@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: S2-2406589 (CR 4821 to TS 23.502)

1. Overall Description:

SA2 would like to thank CT3 for its LS on GPSI and Application Layer ID Mapping.

Regarding the issues raised by CT3:

1)	the Application Layer ID is only added in TS 23.502 clause 5.2.6.11.5 Nnef_ServiceParameter_Get service operation, with GMLC as southbound service consumer which is different from the existing northbound service consumer AF. The other service operations in Nnef_ServiceParameter service is still missing the Application Layer ID;

2)	TS 23.502 clause 5.2.6.11.1	General defines "This service is for allowing external party to provision of service specific parameters which can be used for the UE in 5GS", while the GPSI and Application Layer ID Mapping information is used by GMLC, not transfer to the UE;

3)	TS 23.502 clause 5.2.6.27	Nnef_UEId service including support of HR-SBO Sessions with SUPI exposed to roaming partners i.e., V-NEF as service consumer in southbound, just not yet clearly described and missing V-NEF as service consumer of Nnef_UEId in Table 5.2.6.1-1.).

SA2 has already agreed to add the Application Layer ID in other service operations of Nnef_ServiceParameter service for AF to provision mapping information between GPSI and Application layer ID. The description of the service already includes “This service can also be used to create, update, delete and query Application Layer ID and GPSI mapping for Ranging/SL Positioning defined in TS 23.586 [88].” specific for Application Layer ID and GPSI mapping. And Nnef_UEID service will not be used for ID mapping in Ranging/SL Positioning.

The answer to the question is as follows:

Question:	Among the Nnef_UEId API and Nnef_ServiceParameter API, which one is better to implement the GPSI and Application Layer ID Mapping?

Answer:	Nnef_ServiceParameter is used to implement the GPSI and Application Layer ID Mapping, the AF will use Nnef_ServiceParameter service options to create/update/delete the mapping information between GPSI and Application Layer ID.

SA2 has agreed the attached CR to add clarifications clarify the GPSI and Application Layer ID Mapping issues.

2. Actions:

To CT3:

ACTION:

SA2 would like to ask CT3 to take the above into account and develop the corresponding specifications.

3. Date of Next SA2 Meetings

SA2#164 	19-23 August 2024	Maastricht, NL