3GPP TSG-WG SA2 Meeting #163	S2-2406634

Jeju, Korea, May 27th – 31th, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#8, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_ph2 / Rel-19

Abstract: This paper proposes to add the conclusion section for KI#8 in FS_NG_RTC_Ph2 TR.

# 1. Discussion

Eight solutions were proposed for KI#8: #17, #23, #24, #25, #26, #27, #31, #32.

These solutions have the following similarities:

-	Solutions are based on data channel architecture, a new NF is added to store avatar representation and related data;

-	The avatar representation and related data can be accessed by Avatar-id and user identity;

-	MF/MRF supports avatar representation animation and rending based on audio/video/text or metadata received from UE.

In the following table solutions are classified based on following categories:

-	Which NF connect to DAR;

-	How the UE downloads the avatar representation;

-	Which NF downloads the avatar representation;

-	Which NF controls the avatar service logic;

-	The role of animation/rendering the avatar;

|  | Which NF connect to DAR | How the UE downloads the avatar representation | Which NF downloads the avatar representation | Which NF controls the avatar service logic | The role of animation/rendering the avatar |
| --- | --- | --- | --- | --- | --- |
| Solution #17: Solution for support of IMS avatar communication using data channel | XR AS | over ADC | XR AS | XR AS | UE-A or MF/MRF or UE-B |
| Solution #23: Download avatar representation in bootstrap data channel from avatar repository | DCSF | over BDC | N/A | N/A | N/A |
| Solution #24: Transition Between Avatar and Audio/Video Communication | DCSF | over ADC | DCSF | DCSF | MF/MRF or UE-B |
| Solution #25: Supporting network-based avatar communication by media capability invocation | XR AS | over ADC | MF/MRF | XR AS | MF/MRF |
| Solution #26: Support for Avatars in AR Calls | MF | unclear | unclear | XR AS | MF/MRF or UE-B |
| Solution #27: Early capability negotiation for IMS Avatar Communication | NEF | through NEF | DCSF | DCSF | UE-A or MF/MRF or UE-B |
| Solution #31: Support of IMS DC based avatar communication | DCSF | over BDC | N/A | DCSF | UE-A or MF/MRF or UE-B |
| Solution #32: Solution for support of IMS avatar communication without using data channel | MF | unclear | MF | IMS AS | UE-A or MF/MRF or UE-B |

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-77.

# * * * * First change * * * *

## 8.x Conclusions Related to Key Issue #8

The following conclusions are agreed for supporting IMS Avatar communication:

-	In release 19, avatar communication is implemented only based on the data channel architecture.

-	In release 19, network centric mode and UE-A centric mode, i.e. each UE use their own avatar representation to animate and sent to the peer, is supported. The UE-B centric mode, i.e. each UE use the peer’s avatar representation to animate and sent to the peer, is not supported.

-	A new NF Digital Asset Repository (DAR) is added to store Avatar representation and related data of users who subscribed this service. The DAR belongs to the operator’s network. The DAR provides the service-based interface. There are reference points between DAR and DCSF and between DAR and MF.

-	Avatar ID list is downloaded though bootstrap data channel and can be shared between different applications. DCSF retrieves Avatar ID list from DAR.

-	MF can directly retrieve the avatar representation from DAR using Avatar ID and end user identity if network centric mode is used.

-	The Avatar representation can be animated using audio/video or metadata.

NOTE 1:	The metadata media format and the negotiation procedure for Avatar representation animation is further defined in SA4.

-	The Avatar representation and metadata (such as action and facial expression data) are transmitted between the UE and the IMS network using data channel.

-	When network centric mode is used, DC AS (e.g. XR AS) sends Avatar communication service control instruction to the MF/MRF, the MF/MRF should support media transcoding, e.g. ASR/TTS/SLR/SSL.

NOTE 2:	The content of the instruction sent from DC AS (e.g. XR AS) to MF/MRF should be defined in SA4.

NOTE 3:	Whether the authentication and authorization of avatar representation is needed will be decided by SA3.

# * * * * End of changes * * * *