SA WG2 Meeting S2#163	S2-2406654

May 27 – 31, 2024, Jeju, South Korea	revision of S2-240xxxx

Source:	Nokia

Title:	KI#3: Update on conclusion principles

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes updates to KI3 solution principles to resolve remaining editor notes and to clarify basic solution principles.

# Introduction

KI#3 is about support of IMS DC interworking between DCMTSI and MTSI clients.

# 2	Proposal

It is proposed to add following text to TR 23.700-77.

# * * * * First change* * * *

# 8	Conclusions

## 8.1	Interim conclusion of KI#3

For KI#3 on "IMS data channel interworking with MTSI UE" the following interim conclusions are agreed:

1.		When the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, the originating DCSF is aware that the target UE does not support IMS DC based on the SDP answer from the terminating side and can determine that IMS DC interworking is required for the requested DC application(s).

NOTE 1:	How the DCSF can determine that IMS DC interworking is required is up to implementation.

2.	IMS DC interworking is not a subscription based service.

3.	When the originating DCMTSI UE establishes an application DC, the originating DCSF terminates the data channel media at the MF. The DCSF instructs the IMS AS how to transcode the received data channel media at the MF to MMTel media. The MMTel media are sent by the MF to the terminating UE.

Editor's note:	The details for media transmission from UE-B to UE-A and how the required transcoding is done are FFS.

4.	The MF in the originating network associates data channel media resources with MMTel media resources and transcodes data channel media to MMTel media.

NOTE 2:	How the MF is instructed to transcode data channel media to MMTel media and how the actual transcoding is done in the MF is up to implementation.

# * * * * End of changes * * * *