SA WG2 Meeting S2#163	S2-2406658

May 27 – 31, 2024, Jeju, South Korea	revision of S2-240xxxx

Source:	Nokia

Title:	KI#6: Update on conclusion principles

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes updates to KI6 solution principles to resolve remaining editor notes and to clarify basic solution principles.

# Introduction

KI#6 is about support of standalone IMS Data Channel Sessions.

# 2	Proposal

It is proposed to add following text to TR 23.700-77.

# * * * * First change* * * *

## 8.3	Interim conclusion of KI#6

The following interim conclusions for KI#6 "Support of Standalone IMS Data Channel Sessions" are agreed:

-	Scenarios including establishing IMS session with standalone bootstrap DC, or a combination of standalone bootstrap DC and application DC shall be supported.

-	SIP preconditions are not used in the context of standalone IMS DC session.

-	The application binding information is always included in the SDP offer as specified in Rel-18.

Editor's note:	Whether to support application version compatibility is FFS. Coordination with SA WG4 is needed.

-	When the UE initiates an IMS session with standalone bootstrap DC or standalone application DC:

-	The originating UE generally follows existing procedures to establish bootstrap DC or application DC as specified in TS 23.228 [5] Annex AC with the addition that the UE may only include DC media components when generating SDP offer in initial INVITE request.

-	If the terminating UE has not downloaded the application (either via bootstrap DC or by means outside the scope of 3GPP), or if the terminating UE does not have downloaded an application which is compatible with the application of the originating UE, the terminating UE rejects the session.

-	When the UE initiates an IMS session with combination of standalone bootstrap DC and standalone application DC:

-	The originating UE only include DC media components for both bootstrap DC and application DC when generating SDP offer in initial INVITE request but follows existing procedures to establish bootstrap DC and application DC as specified in TS 23.228 [5] Annex AC.

-	If the terminating UE has downloaded the application as indicated in the SDP offer, the terminating UE responds with the SDP answer for both bootstrap DC and application DC.

-	If the terminating UE has not downloaded the application as indicated in the SDP offer, it can use means outside the scope of 3GPP to download the application before sending the SDP answer.

Editor's note:	How to handle the scenario that the terminating UE has not yet downloaded the application and the UE cannot download the application by means outside 3GPP scope, e.g. how the SDP is negotiated, and when to alert the terminating UE, is FFS.

-	The UE uses re-INVITE request to manage IMS session modifications.

-	The UE and IMS network negotiate the capability of supporting standalone DC during registration procedure, where the UE indicates its capability with a media feature tag in initial and subsequent REGISTER requests and the S-CSCF indicates the network capability with a Feature-Caps header field in the 200 OK to the initial and any subsequent REGISTER request.

-	Subscription for standalone IMS data channel is optional. When subscription for standalone IMS data channel is supported in the network, the subscription information is stored in service data of the IMS data channel service, based on which the IMS AS determines whether to accept the standalone IMS data channel session.

-	Standalone IMS data channel sessions use SIP session timer as per RFC 4028 [20] to avoid hanging resources in the UE and the network. Usage of SIP session timer in IMS is specified in TS 24.229 [10].

-	The standalone IMS data channel session is terminated by the existing session release procedure specified in TS 23.228 [5]. The session release may be triggered by the expiry of the session timer or by sending a SIP BYE request.

-	Standalone IMS data channel sessions and MMTel sessions can co-exist independently.

-	It is assumed that all IMS AS in the network supporting IMS DC are also supporting standalone IMS DC.

# * * * * End of changes * * * *