SA WG2 Meeting #163	S2-2406673

May 20 - 31, 2024 Jeju, South Korea	(Revision of S2-240xxxx)

Source: 	NTT DOCOMO

Title: 	KI #6, Conclusion update for KI#6

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution provides update of conclusion for key issue #6.

# 1	Discussion

This contribution updates the interim conclusions for KI#6.

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-77.

FIRST CHANGE

## 8.3	Conclusion of KI#6

The following conclusions for KI#6 "Support of Standalone IMS Data Channel Sessions" are agreed:

-	Scenarios including establishing IMS session with standalone bootstrap DC, standalone application DC, or a combination of standalone bootstrap DC and application DC shall be supported in this Release.

-	An IMS session with standalone bootstrap DC only is used for downloading the application(s) from local DCSF. The SIP session terminates at the local IMS network of the originating UE. The local DCSF serving the UE instructs IMS AS to terminate the session and reserve only the originating side media resources based on the received DC media components.

-	An IMS session with only application DC media allows to use an application that is already available for terminating UE. If the terminating UE does not have the application, the terminating UE should attempt to download the application upon receiving the SDP offer for the application DC.

-	An IMS session with a combination of standalone bootstrap DC with the remote DCSF and application DC allows the peer UE to use the bootstrap DC with the remote DSCF to download the application from the remote DCSF (if not yet downloaded).

-	If the terminating UE that receives an IMS session for application DC has the application available, the terminating UE responds with the SDP answer that accepts the application DC media and then alerts the user.

-	If the terminating UE that receives an IMS session for application DC has not downloaded the application, terminating UE responds with the SDP answer and rejects the application DC media (with the port of application DC set to zero), and then downloads the application.

-	Once the terminating UE that rejected the application DC media has the application available, the terminating UE updates the session by adding the application DC media to the session and then alerts the user.

-	The peer UE may update the IMS session with application DC media only, and add bootstrap DC with the remote DCSF to the session for downloading the application(s) from the remote DCSF.

-	The terminating UE may update IMS session with a combined standalone bootstrap DC with remote DCSF and application DC and add a bootstrap DC with the local DCSF into the IMS session. After this either party can use either of the DCSFs to download applications and add corresponding application DC media into the IMS session.

- 	At any point of time, after the application DC media has been negotiated for an IMS session, either party may add other media (e.g. audio/video) to the IMS session.

- 	At any point of time, after the IMS DC session with audio/video media as defined in Release 18 has been negotiated, either party may remove the audio/video media from the IMS session.

-	The handling of SIP preconditions for standalone DC IMS session will be addressed during normative phase.

-	The binding information is included in the SDP offer as specified in Rel-18, and may be enhanced if necessary.

-	The UE and IMS network negotiate the capability of supporting standalone DC during registration procedure, where the UE indicates its capability with a media feature tag in initial and subsequent REGISTER request and the S-CSCF indicates the capability with a Feature-Caps header field in the 200 OK to initial and subsequent REGISTER request.

-	Additional subscription for standalone data channel is optional. When additional subscription for standalone data channel is required, the subscription information is stored in service data of IMS data channel, based on which the IMS AS determines whether to accept the standalone data channel IMS session.

End of CHANGES