3GPP SA WG2 Meeting #163	S2-2406693

Jeju, Korea, May 27th – May 31st, 2024           	(revision of S2-240xxxx)

Source: 	Nokia

Title:	Updated conclusions for KI#1

Document for: 	Approval

Agenda Item: 	19.11

Work Item / Release:    FS_UPEAS Ph2 / Rel-19

Abstract of the contribution: Updated conclusions for KI#1.

# Discussion

This pCR provides updated conclusions for KI#1.

-	Interim conclusions for KI#1 include the following:

Editor's note:	It is FFS whether the SMF selection logic of UPF can be based on subscription/and or policy information.

For offering more options for the UPF selection logic of the SMF than only configuration, it is proposed to remove the Editor’s note and include in the conclusions that UDM/UDR subscription data and/or SM Policy data may indicate extended functionalities that are required and/or desired.

# Proposal

It is proposed to capture the following changes in UPEAS Ph2 TR 23.700-63.

*** First change ***

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study..

## 8.1	Conclusions for Key Issue #1: Selection of UPF providing a selected user plane functionality

The following aspects are concluded as principles for normative work:

-	Reusing procedures in clauses 4.17.1 and 4.17.2 of TS 23.502 [3] to support registering the extended UPF functionalities in NRF.

-	The following UPF functionalities are added in the N4 capabilities and UPF NF profile stored in NRF:

-	The functionality of NAT information exposure;

-	Packet Inspection functionality (to differentiate between IP or MAC filter based packet detection, and the packet detection based on other means, e.g. layer 7 DPI);

-	Defining operator configurable parameter in the N4 capabilities and UPF NF profile which can be used for non-standard or partially supported features and configured by operator to extend the baseline UPF capabilities. The format of the parameter is to be determined by stage 3.

-	For example, the operator may configure values for this parameter to represent customized configuration (e.g. hardware accelerators (for example GPU, DPU etc.), Firewall, DDoS Protection, etc.).

-	UDM/UDR per UE subscription data is enhanced to support UPF extended functionalities. UDM/UDR per UE subscription data may indicate extended UPF functionalities that are required and/or desired for a PDU Session.

-	SM Policy data in PCF may indicate UPF extended functionalities that are required and/or desired for a PDU Session.

*** END of changes ***