3GPP TSG-WG SA2 Meeting #163	S2-2406730

Jeju, Korea, May 26th – 31th, 2024	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#2 conclusion proposal

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: This paper proposes conclusion on KI#2 in TR 23.700-49.

1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-49.

# * * * * First change * * * *

8.1	Conclusion for KI#2

The following principles are recommended in normative work for KI #2: Enhancement of EAS and local UPF (re)selection

-	SMF selects local PSA UPF considering N6 delay, when available.

-	N6 delay between L-PSA UPF and EAS may be measured by the SMF after the EAS discovery.

-	The EAS Deployment Information may also include N6 delay for each DNAI and provided to SMF from the AF as specified in TS 23.548. This may be used to select the local PSA UPF before the EAS discovery.

Editor's note:	Whether EAS load can also be used by the SMF/EASDF is FFS.

-	N6 delay between L-PSA UPF and EAS is measured by leveraging existing mechanisms (e.g., defined by IETF, PING, TWAMP, OWAMP, etc.)

-	Interaction between AF and 5GC may be needed to enable the measurement

Editor's note:	Details of such interaction are FFS.

Editor's note:	Whether the NWDAF should be involved is FFS.

# * * * * End of changes * * * *