3GPP TSG-WG SA2 Meeting #163	S2-2406731

Jeju, Korea, May 26th – 31th, 2024	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#3 conclusion proposal

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: This paper proposes conclusion on KI#1in TR 23.700-49.

1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-49.

# * * * * First change * * * *

8.3	Conclusion for KI#3

Interim conclusion:

-	The traffic between local DN and central DN can be routed via GTP-U tunnel between the L-PSA UPF and PSA UPF.

-	The traffic routing rule in the L-PSA UPF and PSA UPF for the GTP-U tunnel should be controlled by SMF, which can be influenced by AF.

-	The GTP-U tunnel for traffic routing between local DN and central DN may be established directly between the L-PSA UPF and PSA UPF, or via existing ULCL/BP, or via a different UPF.

-	The requirement of establishing the GTP-U tunnel for traffic routing between local DN and central DN should be provided by AF.

-	The GTP-U tunnel is released when the PDU Session is released.

-	QoS should be supported for the traffic routed between local DN and central DN.

-	Charging should be supported for the traffic routed between local DN and central DN, any charging issue should be verified with SA WG5.

# * * * * End of changes * * * *