SA WG2 Meeting #163	S2-2406780

# Jeju, Korea, May 27 – May 31, 2024

Source: 	CableLabs

Title: 	KI#4: Evaluation and Conclusions

Document for: 	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This paper provides evaluation and conclusions for KI#4.

# 1	Introduction

This contribution presents evaluation and proposes conclusions for Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG.

This KI aimed to study how 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG and how to provide policy control for the traffic associated with individual non-3GPP devices.

# 2 	Proposal

It is proposed to update TR 23.700-32 as below.

Start of changes, all new text

## 7.	Evaluation

For Key Issue #4, the following criteria are essential for evaluating solutions and thereby conclusions –

- Authentication of non-3GPP devices by the 5GC is not a pre-requisite for concluding any solutions aspects. In certain scenarios, such as in residential environments, it is left up to the household owner to ensure the correct association of a non-3GPP device User Identifier with its QoS policy.

- Solutions should consider the RG/UE scalability issue, whereby requiring a dedicated PDU Session for each non-3GPP device is not feasible.

- Solutions should reuse existing Rel-18 mechanisms for default QoS behavior in order to reduce CP signaling. So, solutions that require CP signaling for each non-3GPP device connecting, whether it requires differentiated QoS or not, are not preferred.

- Frequent updates of RG/UE policy, such as URSP, to support dynamic service differentiation requests are not preferred.

- Solutions that do not consider how the UE/RG/5GC would handle the MAC randomization scenarios with User Identifiers should not be considered.

Next change, all new text

## 8.3	Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

The following conclusion principles are agreed for Key Issue #4:

- Non-3GPP devices can be locally authenticated or via an external AAA server, depending on the device capability.

- It is assumed that the User Identifier bound to each non-3GPP device is known by the UE/5G-RG and the AF. UE/5G-RG keeps the AF up to date (using signalling not standardized in 3GPP) with the connected non-3GPP devices and associated device information.

- The AF, based on the user’s input, provisions a policy for the non-3GPP device in the 5GC. Such policy update will initiate AF-triggered policy update procedure.

- 5GC (UDR) stores the QoS policy associated with a non-3GPP device’s User Identifier.

- In the case of non-3GPP devices sharing a PDU Session, the policy is provisioned in the UE/5G-RG using PDU Session Modification procedures.

- Devices that do not require differentiated QoS shall use the default QoS Flow established using Rel-18 Connectivity Group ID mechanisms.

End of changes