3GPP SA WG2 Meeting #163	S2-2407170

Jeju, South Korea, May 27 – 31, 2024

Source:	Ericsson, Nokia, AT&T, Samsung

Title:	KI#1: Conclusions

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: The contribution proposes conclusions for KI#1.

# Discussion

The following principles should be considered for KI#1 conclusion:

- For Distributed Anchor Point and Multiple PDU Session models, the existing mechanisms shall be used.

- For Session Breakout model,

  - I-SMF based approach shall be considered to reduce impact on central 5GC NFs.

  - I-SMF selection shall be performed by AMF. I-SMF selection mechanism shall be enhanced to support I-SMF for local offloading management, e.g., I-SMF selection for local offloading management is based on the subscription data.

  - EASDF configuration and DNS message handling can be performed by I-SMF.

  - EASDF discovery and selection can be performed by I-SMF.

  - I-SMF shall retrieve EDI from NEF to configure EASDF. EDI management in the I-SMF shall be supported.

  - I-SMF shall receive the local offloading policy from PCF via SMF which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and shall select local UPF(s).

# Proposal

It is proposed to add these changes to TS.  23.700-49

***************** 1st change *****************

## 8.1	Conclusion for KI#1

The following principles are concluded for the normative work.

For Distributed Anchor Point and Multiple PDU Session models, the existing mechanisms shall be used.

For Session Breakout model the following applies:

- I-SMF based approach shall be considered to reduce impact on central 5GC NFs.

- I-SMF selection shall be performed by AMF. I-SMF selection mechanism shall be enhanced to support I-SMF for local offloading management, e.g., I-SMF selection for local offloading management is based on the subscription data.

- EASDF configuration and DNS message handling is performed by I-SMF (instead of SMF).

- EASDF discovery and selection is performed by I-SMF (instead of SMF). When the I-SMF approach is used the SMF does not discover and select an EASDF.

- I-SMF retrieves EDI from NEF to configure EASDF. EDI management in the I-SMF shall be supported. I-SMF can also be preconfigured with the EAS deployment information.

- I-SMF can be configured with or receive the local offloading policy from PCF via SMF which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and shall select local UPF(s).

- SMF may forward AF provided traffic influence information to I-SMF.

***************** End changes *****************