3GPP TSG-WG SA2 Meeting #163 	S2-2407190

May 27 – 31, 2024, Jeju Island, Korea	(Revision of S2-2407171)

Source:	Intel, Huawei, ZTE, CATT, Ericsson, CMCC, Nokia, Samsung, Lenovo

Title:	KI#2: Conclusion update based on NWM discussion

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract: This paper proposes to update the conclusion for KI#2 based on NWM discussion.

# 1. Introduction

This contribution proposes conclusion update for KI#2 based on NWM discussion.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-49.

# * * * * First change * * * *

## 8.2	Conclusion for KI#2

The following principles are concluded for normative work:

-	SMF can select local PSA UPF considering N6 delay, when available.

-	SMF can collect N6 delay measurement from candidate UPF per pair of 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (e.g. EAS/Designated IP (range)) as following:

- SMF determines the candidate UPF(s) according to the candidate DNAI(s) which is based on the list of DNAI(s) received in AF request or EDI.

- SMF determines the measurement protocol based on the protocol supported by UPF and the measurement protocol(s) supported by application side (e.g. Pre-configuration in SMF or via AF request).

- SMF configures the candidate UPF based on the N6 delay monitoring information such as measurement endpoint (e.g. target EAS IP address(es)/Designated IP (range)) per DNAI in the list, the determined measurement protocol and optionally the protocol-specific configuration parameters.

- UPF(s) performs N6 delay monitoring and reports the N6 delay monitoring result to SMF.

-	5GC is not aware of EAS load.

-	N6 delay between 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (i.e. EAS IP address(es)/Designated IP (range)) is measured by leveraging existing mechanisms (e.g., ICMP, TWAMP, OWAMP or other protocols defined by IETF).

-	5GC may enable the N6 delay measurement via AF influence procedure per clause 4.3.6 in TS 23.502[3] or via EAS Deployment Information Management defined in clause 6.2.3.4 in TS 23.548 [5]. When AF influence procedure is used, in Nnef_TrafficInfluence_Create/Update Request, AF can provide an indication of considering N6 delay, a list of DNAI(s), measurement endpoint (e.g. target EAS IP address(es)/Designated IP (range)) per DNAI in the list, the measurement protocol(s) and optionally the protocol-specific configuration parameters.

NOTE 1: For the enabling of SMF to measure N6 delay aspect, whether one mechanism between AF influence and EDI is enough will be determined in normative phase.

NOTE 2: 	For AF-provided delay requirement, it will be determined in normative phase whether a new information element (e.g., end-to-end delay requirement) is needed or the existing User Plane latency requirement can be reused.

NOTE 3: Whether SMF considers N6 delay for EAS selection will be determined in normative phase.

# * * * * End of changes * * * *