SA WG2 Meeting #163	S2-2405845

27 - 31 May, 2024, Jeju, South Korea

SA WG2 Meeting ##162	S2-2403877

15 - 19 April, 2024, Changsha, China

3GPP TSG-CT3 Meeting #133	C3-241301

# Athens, Greece, 26th February – 1st March 2024

Title:	LS on Alternative S-NSSAI related Policy control

Response to:	N/A

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	CT3

To:	SA2

Cc:

Contact person:	Abdessamad El Moatamid

abdessamad.el.moatamid@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

In clause 6.1.3.18 of TS 23.503, the "Network Slice Replacement" event was defined to support the reporting of the network slice replacement event related information to the PCF for the UE, and it is specified that the PCF for the UE subscribes to this Event via AMF and SMF, in a similar way to the existing "SM Policy Association established or terminated" event.

CT3 noticed however that, unlike the "SM Policy Association established or terminated" event:

-	clause 6.1.2.1 of TS 23.503 was not updated to specify in details how the subscription/reporting is carried out, i.e., whether it is exactly the same mechanism as the "SM Policy Association established or terminated" event or there are differences; and

-	the link to the related "Network Slice Replacement" PCRT is missing in clause 6.1.3.5 of TS 23.503.

Therefore, CT3 would like to ask the following questions:

Question 1:	Can SA2 confirm that the "Network Slice Replacement" event is a separate event to the existing "SM Policy Association established or terminated" event as specified in clause 6.1.3.18 of TS 23.503?

Question 2:	Can SA2 clarify in details how the subscription/reporting is performed for the "Network Slice Replacement" event (in a similar way to the provisions of clause 6.1.2.1 and 6.1.3.5 of TS 23.503 for the "SM Policy Association established or terminated" event)?

# 2	Actions

To:  SA2

ACTION: 	CT3 kindly asks SA2 to address the above questions and update their specifications accordingly, if necessary.

# 3	Dates of next TSG CT WG 3 meetings

CT3#134	15th – 19th April 2024	Changsha, Hunan Province, China

CT3#135	27th – 31st May 2024	Hyderabad, India