SA WG2 Meeting #163  	  S2-2405898

27 May-31 May 2024, Jeju island, Korea

Source:	ZTE

Title:	TR 23.700-84: KI#1 Conclusion

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

# 1. Discussion

Interim conclusion for Key issue 1. For the training data collection process, I can accept reuse of positioning procedure entirely.

# 2. Proposal

---------- Start of change(All text new) ----------

8.1 Key Issue #1:  Enhancements to LCS to support Direct AI/ML based Positioning

The following principles are proposed as interim conclusion for KI#1:

- -  LMF perform inference and MTLF perform LMF-side model training. The LMF get the trained model from MTLF.

- -  For training data collection, reuse positioning procedure to find LMF and then LMF send the training data to NWDAF directly.

- - The training data of LMF-side model is determined by RAN WGs.

- -   Training data collection can be conducted per area or per UE(s).

- -   LMF reports the AI capability to NRF and AMF selects LMF base on the AI capability.

- -   Performance monitoring of LMF-side model is done by MTLF.

---------- End of change ----------