3GPP TSG-WG SA2 Meeting #163	S2-2405921

Jeju, Korea, May 27 – May 31, 2024	(revision of S2-231xxxx)

# Title:	Sol#24 Update to solve the EN about the UPF selection

# Source:	China Mobile

# Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract: Propose a new solution to study the enhancements to support store and forward satellite operation for KI #2.

1 Discussion

It is proposed to solve the the following EN,

Regarding “Editor' s note: How the SMF selects on-board UPFs based on satellite ID, ephemeris information is FFS”, it is proposed that SMF selects on-board S&F capable UPFs based on satellite ID, UE location, ephemeris information.

Regarding “Editor's note: How to support the combination of the registration and PDU session establishment is FFS”, it is proposed to support the combination of the registeration and PSU session establishment only when the UE is deregistered,  which means the NAS security context in UE context is stored in the UE and the AMF.

Regarding “Editor's note: How to handle the change of satellite is FFS”, it is proposed let AMF do the target satellite selection based on the UE location and ephemeris information.

2 Proposal

It is proposed to capture the following changes in TR 23.700-29.

FIRST CHANGE

6.24	Solution #24: Enhancement of registration and PDU session establishment to support on-board S&F

6.24.1	Description

The Store and Forward (S&F)  Satellite operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

To support the S&F operation in single satellite case, there could be two deployment options.

Option 1: User plane enabled S&F. Only user plane function is on-board, i.e. the UPF and optionally the EAS could be onboard. User plane on board enabled S&F can support the rapid implementation of delay-insensitive services (CIoT, etc.), and has low requirements for onboard resources. In addition, with the on-board EAS, some service could be provided on the satellite, which could save the feeder link resource, and reduce the transmission delay.

In this case, A 5G system with satellite access supporting S&F Satellite operation shall support mechanisms for a UE to register with the network when the network is in S&F Satellite operation. A S&F registration solution is proposed and will be elaborated in the following clause.

Figure 6.24.1-1: Architecture assumption for User plane enabled S&F

Option 2: 5GC enabled S&F. Both user plane function and part of the control plane are on-board, i.e. the UPF, AMF, SMF, UDM/AUSF and optionally the EAS could be onboard. 5GC enabled S&F has requirements for onboard resources, and control signaling exchange on the satellite can save feeder link resources.

In this case, most 5GC service could be provided on-board with minor enhancement. The SMF is supposed to have the S&F policy about the S&F operation. Detailed enhancement will be elaborated in the following clause.

Figure 6.24.1-2: Architecture assumption for 5GC enabled S&F

6.24.2	Procedures

6.24.2.1	Option 1: Procedures of User plane enabled S&F

To support mechanisms for a UE to register with the network when the network is in S&F Satellite operation, this solution proposes the UE request the registration and PDU session establishment into single procedure. Following enhancement are supposed to be supported by (R)AN.

1.	When the S&F operations are supported and indicated in the UE request, (R)AN shall store the received UE registration and PDU session establishment request. RAN is aware of the ephemeris information, by configuration.

2.	Trigger the the feeder link/service link establishment when they are available. Upon the feeder link/service link is available, send out the stored UE registration and PDU session establishment request.

6.24.2.1.1	Procedures of registration in S&F mode

Pre-condition：

- Every satellite has RAN and UPF onboard.

- For this solution, the UE is considered to have registered to the 5G network.

Figure 6.24.2.1.1-1: Procedures of registration in S&F mode

1.	UE sends the PDU session establishment request to (R)AN. The request message includes the indicator of the S&F PDU session establishment.

The indicator of the S&F PDU session establishment indicates the UE support the S&F PDU session establishment and wants to perform a S&F PDU session establishment.

Editor's note:	How to support security mechanism is FFS, and need to coordinate with SA3.

2.	When the UE provides the S&F PDU session establishment request, the (R)AN stores the UE request received.

3.	The (R)AN sends a S&F PDU session establishment notification to the UE. The notification would inform the UE the PDU session establishment is in the S&F mode, indicating the response will be delayed.

Instead of AMF trigger the paging, UE would be in RRC_inactive state, (R)AN store the UE context, and send the RAN Notification area and RAN Notification Area Update timer to UE. The RAN Notification Area Update timer is alignment with the ephemeris information, i.e. UE will send the periodic RAN Notification Area Update to (R)AN at the time that the service link is available again.

The "RRC Inactive Assistance Information" could be pre-configured or provided by AMF in N2 activation procedure. An S&F mode capability information may be provided in the "RRC Inactive Assistance Information" to help NG RAN's decision whether the UE can be sent to RRC_INACTIVE state, including the indication of the supporting the S&F mode, and optionally, the S&F triggered RRC Inactive timer.

Or, if the UE is aware of the ephemeris information, the UE may trigger the transfer to the RRC_INACTIVE state based on the ephemeris information.

4.	Based on the ephemeris information, or the pre-configuration, (R)AN sends the stored request to AMF upon the feeder link is available.

5.	Complete the PDU session establishment procedures on the ground as defined in clause 4.3.2.2 step 2-12 of TS 23.502 [3].

With the indication of the S&F PDU session establishment, the AMF is aware that the UE is in S&F mode, AMF may, based on configuration and ephemeris information, determine whether the current satellite can cover the user's location for the next round when the service link is available again. If not, AMF selects a target RAN on second satellite based on the UE location, ephemeris information. Then the AMF sends the satellite ID, UE location to the SMF.

AMF may request and store the UE context from the source RAN, and send it to the target RAN.

The SMF selects on-board S&F capable UPF based on satellite ID, UE location, ephemeris information. The SMF activates the on-board UPF buffering in the N4 session signaling.

- The AMF determine if the next satellite serving the UE would change, if it is going to change, the source (R)AN transfer the UE context to the target (R)AN via AMF on the ground or ISL(if available).

- 7.

The AMF sends the PDU Session Establishment Accept to the (target) (R)AN. The (target) (R)AN stores the information received from AMF that will send to the UE. The (target) (R)AN stores the UE context and acknowledges to the AMF.

The S&F Extended connected timer for N2 could be provided by AMF to indicate how long the N2 session AMF needs to be maintained.

8.	When the timer is triggered, which means the service link is available, to make UE trigger the periodic RAN Notification Area Update procedure to connect with (R)AN. the (R)AN send out the stored PDU Session Establishment Accept information to UE. And (R)AN forwards the NAS message (PDU Session ID, N1 SM container (PDU Session Establishment Accept)) provided in step 12 to the UE.

9.	(R)AN stores the N2 PDU session response, which will be send to the 5GC when the feeder link is available.

10.	Optionally the Uplink Data will be stored in the on-board UPF, wait for the next service link available time.

To support the service continuity, SMF may be responsible for the session context transfer between the source UPF and target UPF.

6.24.2.2	Option 2: Procedures of 5GC enabled S&F

6.24.2.2.1	Enhancement of the registration procedures

When the AMF, SMF, UDM/AUSF are all on-boarded, the registration procedures follow Registration procedures defined in clause 4.2.2.2 of TS 23.502 [3]. The differences are:

-	The step 16 could be skipped, since the AM policy could be predefined.

-	After the on-board registration completion, the on-board UDM may send the stored UE subscribe data to the ground UDM for backup.

-	The capability of NF(s) supporting the S&F mode need to be indicated.

6.24.2.2.1	Enhancement of the session management procedures

When the AMF, SMF, UDM/AUSF are all on-boarded, the registration procedures follow Registration procedures defined in clause 4.2.2.2 of TS 23.502 [3]. The differences are:

-	The step 7 and/or step 9 could be skipped, since the SM policy could be predefined.

-	The capability of NF supporting the S&F need to be indicated.

6.24.2.2.3	Subscription data retrieve procedures

When the UE’s subscription data is not available in the satellite, the on-board UDM would request the subscription data from the ground UDM.

Pre-condition：

- The onboard UDM has the access information, address information of the ground UDM from OAM or pre-configuration, and the link between the onboard and ground UDMs is available.

- The on-board UDM sends out the subscription data retrieve request to the ground UDM, with the Security parameters, interested UE identifier, requested subscription data type.

- The ground UDM checks whether the on-board UDM is authorized. If authorized, then the ground UDM retrieves the requested subscription data based on UE identifier, requested subscription data type.

3.	The ground UDM sends a subscription data retrieve response including the retrieved subscription data to the on-board UDM.

6.24.2.2.4	Subscription data update procedures

The ground UDM would periodically push updated subscription data to the on-board UDM.

Pre-condition：

- The ground UDM has the access information, address information of the on-board UDM from OAM or pre-configuration, and the link between the onboard and ground UDMs is available.

- The ground UDM as been pre-configured to push the updated subscription data to the on-board UDM periodically.

- Based on pre-configuration, the ground UDM sends out the subscription data update request to the on-boaed UDM, with the Security parameters, updated subscription data.

- The on-board UDM checks whether the ground UDM is authorized. If authorized, then the on-board UDM update the subscription data.

3.	The on-board UDM sends a subscription data update response to the ground UDM.

6.24.3	Impacts to Services, Entities and Interfaces

User plane on-board:

-	SMF: selection of UPF based on the satellite ID and ephemeris information.

-	(R)AN: Store the request from the UE in S&F mode.

-	AMF: selection of target (R)AN based on the UE location and ephemeris information.

The timers related to registration procedure and PDU session establishment procedure may need to be updated.

Control plane and user plane on-board:

-	UDM: subscription data retrieve and update.

The timers related to registration procedure and PDU session establishment procedure may need to be updated.

End of CHANGES