SA WG2 Meeting #163	S2-2405956

Jeju, Korea, May 27 – May 31, 2024	(was S2-24XXXXX)

Source:	vivo

Title:	KI#1, Interim conclusions for AQP

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: This paper proposes interim conclusions for KI#1 AQP.

1	Proposal

It is proposed to include the following changes in TR 23.700-70.

* * * * Start of Changes – All New Text * * * *

8.X	Key Issue #1 Interim conclusions

The following conclusion principles are agreed for key issue #1 QoS Notification.

- When the original QoS profile include UL and/or DL PDU Set QoS parameter, the Alternative QoS profile may include the correspond PDU Set QoS parameters. the QoS parameters in Alternative QoS profile may be one or more of the following: UL/DL GFBR, PER, PDB, UL/DL PSDB, UL/DL PSER.

- When the original QoS profile doesn’t include some PDU Set QoS parameter(s), the alternative QoS profile does also not include the corresponding PDU Set QoS parameters(s).

- The RAN indicates to the CN that PDU Set handling supporting or not in per direction.

- If QoS notification is determined as “GFBR cannot guaranteed”, based on the notification control, the NG RAN determines any AQP can be fulfilled or not in priority order based on the PDU Set handling support or not in both directions.

  - If the PDU Set handling is supported in UL and/or DL, in the corresponding direction, instead of PDB and PER, the PSDB and PSER in the AQP are used to determine the AQP can be fulfilled or not.

  - If the PDU Set handling is not supported in UL and/or DL, in the corresponding direction, the received PDU Set QoS parameters in the AQP are ignored and the PDB and PER in the AQP are used to any AQP can be fulfilled or not.

- Based on the received the reference of the fulfilled Alternative QoS profile and PDU Set handling support indications in UL and DL, the 5GC and the AF understands a set of QoS parameters of the Alternative QoS profile is fulfilled or not.

  - If the PDU Set handling is supported in UL and/or DL, the PDB and PER in the corresponding direction are not considered in the QoS parameter set.

  - If the PDU Set handling is not supported in UL and/or DL, the PSDB and PSER in the corresponding direction are not considered in the QoS parameter set.

* * * * End of Changes – All New Text * * * *