SA WG2 Meeting #163	S2-2405974

Jeju, Korea, May 27 – 31, 2024	(revision of S2-24xxxxx)

Source:	LG Electronics

Title:	KI#2: Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes Evaluation and Conclusion for KI#2.

Discussion

For KI#2, evaluation and conclusion are needed.

The following solutions for KI#2 are described in TR 23.700-59v0.3.0:

-	Solution #4: Network-supported Tactical Deconfliction

-	Solution #5: Support Network-assisted DAA with Existing 5GC Services

-	Solution #6: UTM requests information used for DAA

Proposal

It is proposed to agree the following changes into TR 23.700-59.

* * * * Start of 1st Change * * * *

All New Texts

7.2	Overall Evaluation for Key Issue #2: Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information

The following solutions for KI#2 are described:

-	Solution #4: Network-supported Tactical Deconfliction

-	The solution proposes an AIML-based localized USS/UTM function tailored specifically to provide Network-based DAA services to UAVs and UTM. This function is identified as Localized DAA Server (LDS) for tactical deconfliction. The LDS is a functionality separate from USS/UTM due to the different service it provides, and the fact that LDS may be provided by the MNO.

-	Solution #5: Support Network-assisted DAA with Existing 5GC Services

-	This solution proposes to utilize the GMLC service on Ranging/Sidelink Positioning location and Relative Proximity predictions on collision generated at NWDAF to support Network-assisted DAA. The calculation of relative distance between two UAVs may be performed by GMLC or USS.

-	Solution #6: UTM requests information used for DAA

-	This solution proposes that UTM requests Sidelink positioning location results, List of Aerial UEs in a geographical area and NWDAF analytics from 5GC used for DAA. The UTM requests are made via NEF/UAS NF or directly to 5G NF.

* * * * Start of Next Change * * * *

All New Texts

8.2	Conclusion for Key Issue #2: Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information

It is agreed to select Solution#5 as basis for normative work with the following consideration:

-	Taking aspects proposed in Solution#6 except USS/UTM directly requesting service from 5G NFs into account is needed.

* * * * End of Changes * * * *