SA WG2 Meeting #163	S2-2405995

# 27 – 31 May 2024, Jeju, South Korea

Source: 	NTT DOCOMO

Title: 	KI#3, Interim conclusion

Document for: 	Approval

Agenda Item: 	19.15 (FS_AIML_CN)

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion for KI#3.

# 1	Discussion

Based on the solutions documented in TR 23.700-84, this paper proposes a set of principles to be considered in the normative work for KI#3.

# 2 Proposal

It is proposed to update TR 23.700-84 according to the following text.

********** First Change (All new text) **********

# 8	Conclusions

## 8.X	Interim Conclusions for Key Issue #3: NWDAF-assisted policy control and QoS enhancement

It is agreed to consider the following principles for the normative work:

- NWDAF may provide assistance for policy control and QoS.

- In the current release of the specification, PCF is the only consumer of the NWDAF assistance.

- To provide the assistance for policy control and QoS, NWDAF shall not be aware of PCF internal logic.

- The NWDAF assistance for policy control and QoS may include:

- 	Analytics of QoS parameters (can be provided by collecting the information of applied QoS parameters) and their corresponding evaluation (can be provided via enhancement, if needed, of the existing analytic IDs such as observed user experience, QoS sustainability, etc.) for a particular situation determined by the Analytics Filters.

-	Evaluation (in terms of user experience, QoS sustainability, etc.) of QoS policy/parameters provided by the PCF as an input.

Editor's Note: Providing other assistance information by NWDAF is FFS.

- The PCF may subscribe to the NWDAF assistance and consider it in determining PCC rules based on the operator's policy.

********** End of Changes **********