SA WG2 Meeting #163	S2-2406012

27-31, May 2024, Jeju, South Korea

Source:	OPPO

Title:	FS_UIA_ARC KI#1 Conclusion

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This contribution provides conclusion for KI#1 of the FS_UIA_ARC.

# Discussion

This contribution proposes updates to resolve the following Editor’s note in clause 8.1 Key Issue#1: identifying the Human User of a Subscription

Editor's note:	Where the User Identity Profile is stored is FFS.

Editor's note:	Whether there can be more than one User identifier in the User identity Profile is FFS. The IE in the User Identity Profile is FFS.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-32 V0.3.0.

* * * * Start of Changes * * * *

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

Based on KI#1 solution evaluation in clause 7.X and NWM discussion, the following bullets are agreed as conclusion principles for KI#1:

Editor’s note: 	The above conclusion principles are only tentatively agreed.

-	The User Identifier format is NAI.

-	The User Identity Profile may contain:

-	one or more User Identifiers;

- 	QoS information;

- 	DNN/S-NSSAI that the user is allowed to access;

-	a list of linked subscriptions.

-	The User Identity Profile is stored in the UDR.

-	A trusted AF or OAM can be used to link/unlink a user identifier with a subscription.

-	The User Identifier is unique within PLMN and can be linked/unlinked by a trusted AF or OAM procedure with a 5GS subscription identified by a SUPI.

- 	PCC rules are updated when a user becomes active with a subscription.

- 	PCF can receives information from the user profile to adjust PCC rules.

-	A linked user becomes active with a subscription by providing a user identifier in a NAS message.

* * * * End of Changes * * * *