SA WG2 Meeting #163	S2-2406018

27-31, May 2024, Jeju, South Korea

Source:	OPPO

Title:	FS_UIA_ARC KI#4 Conclusion

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This contribution provides conclusion for KI#4 of the FS_UIA_ARC.

# Discussion

Based on solution evaluation in clause 7.X and NWM discussions, the following conclusions for KI#4 of FS_UIA_ARC are proposed to proceed for normative work. It is also proposed to determine the term for the identifier of the non-3GPP devices connecting behind UE/5G-RG used in normative work, temporarily referred to as “the identifier” in this document.

Storage and Usage of the Identifier:

- The identifier is stored in UDM/UDR and is used by the network to control the traffic to/from UE/5G-RG when the traffic is associated with the non-3GPP devices.

- Each instance of the identifier has an association with certain QoS settings, such as 5QI.

- The UE/5G-RG obtains a list of the identifiers in NAS-MM signalling and the UE/5G-RG is responsible for binding them with trusted non-3GPP devices. The NAS signaling messages (e.g., Registration Request) are modified to include the identifier. The AMF processes these messages and stores the identifier in the UE context.

- The UE/5G-RG includes the identifier in NAS-SM signalling messages to the 5GC. The 5GC then uses these identifiers to apply device-specific policies and manage traffic. During PDU session establishment or modification, the UE/5G-RG includes the identifier in the NAS message, ensuring that the 5GC is aware of the specific device generating the traffic.

Policy Retrieval and Enforcement:

- The PCF retrieves the identifier-specific policies from the UDR based on the identifier. Policies may include QoS parameters, data caps, and service levels, etc.

- Policy control for the traffic of individual non-3GPP devices can be triggered by existing procedures (PCRT, URSP, NAS-SM) enhanced with the identifier support.

Maximum number of simultaneously active identifiers:

- The UE/5G-RG, with the knowledge of the identifier as well as the allowed maximum number of simultaneously active identifiers, enforces the restriction on Device User ID.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-32 V0.3.0.

* * * * Start of Changes * * * *

## 8.4	Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

The following conclusions for KI#4 of FS_UIA_ARC is proposed to proceed for normative work. It is also proposed to determine the term for the identifier of the non-3GPP devices connecting behind UE/5G-RG used in normative work, temporarily referred to as “the identifier” in this document.

Storage and Usage of the Identifier:

- The identifier is stored in UDM/UDR and is used by the network to control the traffic to/from UE/5G-RG when the traffic is associated with the non-3GPP devices.

- Each instance of the identifier has an association with certain QoS settings, such as 5QI.

- The UE/5G-RG obtains a list of the identifiers in NAS-MM signalling and the UE/5G-RG is responsible for binding them with trusted non-3GPP devices. The NAS signalling messages (e.g., Registration Request) are modified to include the identifier. The AMF processes these messages and stores the identifier in the UE context.

- The UE/5G-RG includes the identifier in NAS-SM signalling messages to the 5GC. The 5GC then uses these identifiers to apply device-specific policies and manage traffic. During PDU session establishment or modification, the UE/5G-RG includes the identifier in the NAS message, ensuring that the 5GC is aware of the specific device generating the traffic.

Policy Retrieval and Enforcement:

- The PCF retrieves the identifier-specific policies from the UDR based on the identifier. Policies may include QoS parameters, data caps, and service levels, etc.

- Policy control for the traffic of individual non-3GPP devices can be triggered by existing procedures (PCRT, URSP, NAS-SM) enhanced with the identifier support.

Maximum number of simultaneously active identifiers:

- The UE/5G-RG, with the knowledge of the identifier as well as the allowed maximum number of simultaneously active identifiers, enforces the restriction on Device User ID.

* * * * End of Changes * * * *