SA WG2 Meeting #163	S2-2406025

27 - 31 May, 2024, Jeju, South Korea

# Title:	Solution#24: Enhancements to extend green subscription information towards PCC

# Source:	OPPO

# Document for:	Approval

Agenda Item:	19.4

Abstract:	This document proposes to extend Solution#24 to pass green subscription information towards PCC.

# 1. Introduction/Discussion

Within the scope of the study on Energy Efficiency and Energy Saving, it is proposed to extend the Solution#24 and include passing of green subscription information towards PCC so it can also influence PCC related decisions.

# Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66.

# * * * * First change * * * *

## 6.24	Solution #24: Subscription enhancements for energy savings in RAN

### 6.24.1	Key Issue mapping

This solution is related to the KI#2: Subscription and policy control to support energy efficiency and energy saving as service criteria. This solution addresses green subscription use cases like the use case described in clause 5.1 of TR 22.882 [5].

A green subscription is a form of subscription where the user that allows the operators to save resources by offering lower performance for best effort communication when this can save resources e.g. energy usage. In exchange the user may get a lower subscription fee and periodic estimation on how much energy and/or carbon emission the user has contributed to saving.

Editor's note:	GSI is one example of possible indicators and name. It´s used in this solution to outline the principles of the solution. A as part of the evaluation and conclusions another variant of the indicator may be agreed.

### 6.24.2	Functional Description

There are no architecture updates needed for this solution and existing procedures are reused.

A Green Subscription Indication (GSI) is added to the Access and Mobility Subscription information. The AMF retrieves the subscription information from UDM during registration and passes the GSI to NG-RAN which stores it in the NG-RAN UE Context. It's up to implementation exactly how the NG-RAN use the GSI. It's as an additional input that NG-RAN can use to save resources when there are constraints for best effort services with the understanding that the GSI indicates that the subscriber has accepted that performance may decrease if it saves resources for the operator.

Additionally, in case access and mobility related policy control is enabled, the AMF may report the GSI to the PCF. PCF, based on this new set of GSI information, may make its e.g. SM, AM, BDT, PDTQ and UE policy decisions. PCF may also transfer the GSI indication to the CHF via SMF, by e.g. using specific charging key or charging key/service identifier combination or introducing a new green subscription indication parameter as a part of PCC rule information. CHF, once getting this information, may apply a different charging for the subscriber/subscriber’s session e.g. make it free of charging in certain hours with peak energy consumption, make the subscriber pay less for a certain services etc.

Editor's note:	The method of GSI transfer (e.g. existing or new parameter) is FFS.

All energy savings are not necessarily measurable e.g. energy saving from equipment or capacity that did not need to be deployed. It's left to the operator's discretion to estimate how much energy saving and/or carbon savings the aggregate of all GSI subscribers have resulted in. This information could e.g. be provided to the subscribers the monthly bill. No impacts on SA2 specifications are expected for the savings estimations.

### 6.24.3	Procedures

The AMF retrieves the subscription information from UDM during registration and if changed the GSI is passed to NG-RAN in the UE context setup or modification as described in step 14a-c in General Registration procedure (clause 4.2.2.2.2 of TS 23.502 [3]).

NOTE:	The GSI may also be provided in additional procedures.

### 6.24.4	Impacts on existing services, entities and interfaces

AMF:

-	Retrieving the GSI from the UDM and providing it to RAN and to PCF.

UDM:

-	Storing and providing the GSI.

NG-RAN:

-	Implementations may take the GSI into consideration to save resources consumed on best effort traffic.

PCF:

-	Receiving the GSI from AMF and taking GSI into account while making policy decisions. Sending GSI to SMF.

SMF:

-	Storing and providing the GSI to CHF.

CHF:

-	Taking GSI into account while making charging and billing decisions.

Editor's note:	The method of GSI transfer (e.g. existing or new parameter) is FFS.

Editor's note:	The part of the solution requiring passing of GSI to CHF requires confirmation by SA5.

# * * * * End of changes * * * *