3GPP TSG-SA WG2#163	S2-2406030

Jeju, South Korea, 27 – 31 may 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	KI 1: Conclusion

Document for:	Discussion/Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes some principles for the conclusion of KI#1.

# 1.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-49:

# >>>>BEGINNING OF CHANGES<<<<

## 8.1	Conclusion for KI#1: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs.

The following principles are concluded for the normative work:

-	Only I-SMF based offloading is moved to the normative phase. L-SMF based offloading is not selected for the normative phase.

-	For Distributed Anchor Point and Multiple PDU Sessions connectivity models (see TS 23.548 [5] clause 4.3), the I-SMF can be used as per Rel-18 and no further enhancements are needed.

-	For the Session Breakout connectivity model (see TS 23.548 [5] clause 4.3):

-	The AMF selects an I-SMF capable of supporting local offload management based on subscription data and UE location information.

-	The selected I-SMF retrieves the EAS Deployment Information (EDI) from the NEF and performs EASDF configuration.

-	The selected I-SMF retrieves the local offloading policy which indicates IP range(s) and/or FQDN(s) routable to the local part of the DN and selects the local UPF(s).

# >>>>END OF CHANGES<<<<