3GPP TSG-WG SA2 Meeting #163	S2-2406070

Jeju, Korea, May 27th – May 31st, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion for KI#1: Selection of UPF providing a selected user plane functionality

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: This contribution proposes conclusion for KI#1.

1. Discussion

As captured in Sol#2, the following bullets can be concluded as principles for normative work:

-	Reusing procedures in clause 4.17.6 of TS 23.502 [3] to support SMF provisioning of UPF via the NRF by providing the extended UPF functionalities above in the request.

NOTE:	The N4 Node Level Procedures as defined in clause 4.4.3 of TS 23.502 [3] can also be reused to exchange the supported extended UPF functionalities.

-	Reusing procedures in clause 4.17.4 of TS 23.502 [3] to support UPF discovery via the NRF by providing the extended UPF functionalities above in the request.

In addition, the following EN is removed, since the selection of enhanced UPF can already be supported by using specific DNN, S-NSSAI combination.

Editor's note:	It is FFS whether the SMF selection logic of UPF can be based on subscription/and or policy information.

2. Proposal

It is proposed to capture the following changes to TR 23.700-63.

# * * * * Start of changes * * * *

8.1	Conclusions for Key Issue #1: Selection of UPF providing a selected user plane functionality

The following aspects are concluded as principles for normative work:

-	Reusing procedures in clauses 4.17.1 and 4.17.2 of TS 23.502 [3] to support registering the extended UPF functionalities in NRF.

-	Reusing procedures in clause 4.17.6 of TS 23.502 [3] to support SMF provisioning of UPF via the NRF by providing the extended UPF functionalities in the request.

NOTE:	The N4 Node Level Procedures as defined in clause 4.4.3 of TS 23.502 [3] can also be reused to exchange the supported extended UPF functionalities.

-	Reusing procedures in clause 4.17.4 of TS 23.502 [3] to support UPF discovery via the NRF by providing the extended UPF functionalities in the request.

-	The following UPF functionalities are added in UPF profile stored in NRF:

-	The functionality of NAT information exposure;

-	Packet Inspection functionality (to differentiate between IP or MAC filter based packet detection, and the packet detection based on other means, e.g. layer 7 DPI);

-	Defining operator configurable parameter in the N4 capabilities and UPF NF profile which can be used for non-standard or partially supported features and configured by operator to extend the baseline UPF capabilities. The format of the parameter is to be determined by stage 3.

-	For example, the operator may configure values for this parameter to represent customized configuration (e.g. hardware accelerators (for example GPU, DPU etc.), Firewall, DDos Protection, etc.).

# * * * * End of changes * * * *