3GPP TSG-WG SA2 Meeting #163	S2-2406071

Jeju, Korea, May 27th – May 31st, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion for KI#2: Enhancements on UPF information exposure

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: This contribution proposes conclusion for KI#2.

1. Proposal

It is proposed to capture the following changes to TR 23.700-63.

# * * * * Start of changes * * * *

8	Conclusions

8.2	Conclusions for Key Issue #2: Enhancements on UPF information exposure

The following aspects are concluded as principles for normative work:

-	NEF/NWDAF can use the IP address to subscribe directly to the UPF for data collection.

NOTE:	The restrictions for subscribing via the SMF (as defined in clause 5.8.2.17 of TS 23.501 [2]) should be kept, e.g., QoS Monitoring needs to be subscribed via SMF.

-	Enhance UPF event exposure service to provide the NATed UE public IP address and Port, based on the private UE address (UE IP address assigned by 5GC for the PDU session).

-	When the UPF relocation occurs, the source UPF can send the subscription termination indication and optionally the collected data to the consumer based on the last notification indication from SMF.

# * * * * End of changes * * * *