3GPP TSG-WG SA2 Meeting #163	S2-2406100

Jeju, Korea, May 27th – 31st, 2024

Source:	Huawei, HiSilicon

Title:	KI#4: Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract: This document provides the evaluation and conclusion for KI#4 regarding Efficient mobility and service continuity.

1. Discussion

This document tries to add the evaluation and conclusion for KI#4.

KI#4 in TS 23.700-06 is for Efficient mobility and service continuity when served by MWAB:

5.3 Key Issue #4: Efficient mobility and service continuity when served by MWAB … The following aspects need to be studied for UEs served by the MWAB in the case of mobility in the scenarios A and B: - Whether and how to enhance current procedures of mobility and service continuity for a UE. The following aspects need to be considered in potential solutions: - how to reflect the change of MWAB serving cell or location in the mobility management of the UEs served by the MWAB. - how to efficiently manage the mobility of the UEs served by the MWAB, when 5GC node change is necessary. - how to manage the RAN-CN interfaces. NOTE 2: Mechanisms related to mobility management and service continuity have RAN dependency and should align with the progress of RAN WGs.

By now there are three solutions related to KI#4, namely:

- Solution #8: Provisioning of efficient mobility and service continuity when served by MWAB;

- Solution #9: UE mobility handling due to MWAB mobility;

- Solution #10: New solution to address mobility aspects of an MWAB;

In Solution #8, the following issues are addressed:

-	MWAB mobility:

-	Interaction with serving PLMN OAM, to configure the operation of MWAB-gNB;

-	Cell ID/TAC management by MWAB-gNB in case of mobility;

-	Management of NGAP connection with AMF;

-	UE mobility aspects:

-	UE mobility between a fixed cell and the MWAB cell;

-	UE mobility between MWAB cells;

-	UE mobility when moving together with the MWAB cell; The detailed procedure is FFS.

In Solution #9, the main content includes:

-	MWAB mobility:

-	Interaction with serving PLMN OAM, to configure the operation of MWAB-gNB;

-	Cell ID/TAC management by MWAB-gNB in case of mobility;

-	BH connection handling (same UPF is kept or changed);

-	Management of NGAP connection with AMF;

-	UE mobility aspects:

-	UE mobility when moving together with the MWAB cell (Existing mobility procedure);

-	UE mobility when NOT moving together with the MWAB cell. This solution contains two alternatives in case of AMF change, one is multiple virtual cells, the other is intra cell + UE initiated TAU. Both need to wait for RAN3 confirmation.

Solution #10 suggests to use a dedicated SNPN ID to control the UE to access the MWAB-gNB.

-	MWAB mobility:

-	Management the connection with AMF/UPF (with the connected 5GC node changed/unchanged);

-	UE mobility aspects.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

7.4	KI#4 Evaluation

By now there are three solutions related to KI#4, namely:

-	Solution #8: Provisioning of efficient mobility and service continuity when served by MWAB;

-	Solution #9: UE mobility handling due to MWAB mobility;

-	Solution #10: New solution to address mobility aspects of an MWAB;

Among the solutions for KI#4, the following aspects are addressed:

-	MWAB mobility:

-	Interaction with serving PLMN OAM, to configure the operation of MWAB-gNB (Solution #8, #9);

-	Cell ID/TAC management by MWAB-gNB in case of mobility (Solution #8, #9, #10);

-	Management of NGAP connection with AMF(Solution #8, #9, #10);

-	BH connection handling (Solution #9);

-	UE mobility aspects:

-	UE mobility between a fixed cell and the MWAB cell (Solution #8, #9);

-	UE mobility between MWAB cells (Solution #8, #9);

-	UE mobility when moving together with the MWAB cell (Solution #8, #9).

Summary of the above-mentioned solutions can be found in Table 7.4-1.

Table 7.4-1:  Summary of the solutions of KI#4

| Solutions | UE impact | Highlighted OAM configured information | Management on N2 connection | TAC/Cell ID management | Management of N3 connection | Management on BH connection handling | UE mobility |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solution #8 | No | Cell ID, TAC, AMF address, possibly associated with validity area. | Parameters are (re-) configured by OAM; MWAB-gNB manages NGAP connection as per OAM configuration | MWAB-gNB chooses the TAC/Cell based on physical location and the received parameters. | Reuse current HO procedure | Not mentioned. | No changes UE mobility between a fixed cell and the MWAB cell/ between MWAB cells. FFS on UE mobility when moving together with the MWAB cell |
| Solution #9 | No | Cell ID, TAC, gNB ID | Parameters are (re-) configured by OAM; MWAB-gNB manages NGAP connection as per OAM configuration | MWAB-gNB reports new location information and OAM updates configuration of MWAB-gNB | Reuse current HO procedure | BH UPF retained or not retained have different SSC requirements on BH PDU Session. | No changes when the UEs do not move together with the MWAB. UE uses HO/TAU in case of mobility together with MWAB |
| Solution #10 | No | Not mentioned | AMF determines to trigger AMF relocation | Not mentioned. | Reuse the current HO steps | Not mentioned. | Using Handover procedure, the detail is FFS |

# * * * * Second change * * * *

8.4	KI#4 Conclusion

The following conclusions are reached as baseline for normative work to support the Efficient mobility and service continuity when served by MWAB:

-	Configuration of the MWAB-gNB (e.g., Cell ID/TAC, gNB ID, AMF address, etc.) are managed by OAM of the PLMN it serves, based on the geographical location information of the MWAB.

NOTE 1:	The details of the MWAB-gNB and OAM interactions will be coordinated with RAN3 or SA5.

-	The existing mobility procedures can be reused to handle UE’s mobility to/from a MWAB-gNB without enhancements.

NOTE 2:	Any potential additional enhancements will be led by RAN WGs.

-	No UE impact to support mobility involving a MWAB.

-	In case if Cell ID broadcasted by MWAB-gNB is to be changed, MWAB-gNB updates/establishes N2 connection with the AMF.

Editor's Note:	If inter-gNB handover is needed, whether it is necessary/feasible to instantiate multiple virtual cells by the same MWAB-gNB needs to wait for RAN3 confirmation.

-	In case if TAC broadcasted by MWAB-gNB is to be changed, MWAB-gNB updates/establishes N2 connection with the AMF, and AMF relocation might be performed during mobility registrations triggered by the UEs served by a MWAB.

Editor's Note:	Whether TAC/Cell ID broadcasted by MWAB-gNB will be changed when MWAB moves requires the input from RAN WGs.

# * * * * End of changes * * * *